# Simple Meimad Guide for Haas NGC Postprocessor Programmers

Meimad Production Planner

Beginner-friendly Cimatron GPP/GPP2 and SolidCAM integration examples

| Document | Simple Meimad Guide for Haas NGC Postprocessor Programmers |
| --- | --- |
| Audience | Cimatron GPP/GPP2, SolidCAM, and first-time postprocessor programmers |
| Status | Writer guide; Machine-specific macro numbers and cycle DPRNT require approved configuration |
| Revision | 1.6 |
| Date | 1 September 2026 |
| Source | docs/haas-ngc-postprocessor-and-macro-specification.md |

Safety notice

Start with the required G65 verification hook. Do not enable CST/CEN until Meimad approves the Machine-specific event sequence variable and physical cycle boundaries. Inspect the generated NC text before release.

Contents

What you need to change	3

1. The most important line	3

2. Where the line goes	3

3. Values the post needs	4

4. Header comments	4

5. Simple post logic	4

6. Special instructions for SolidCAM post writers	5

Where to add the hook	5

SolidCAM properties	5

SolidCAM multi-job and subprogram warning	6

SolidCAM delivery check	6

SolidCAM GPPL example	6

7. Special instructions for Cimatron post writers	7

7.1 First identify which Cimatron post you are using	7

7.2 Understand the output point	7

7.3 Beginner workflow: make and prove the required change	8

7.4 What each original GPP line means	8

7.5 A simpler GPP2 example: verification is always required	8

7.6 Common placement mistakes	9

7.7 Split output: every NC file needs a different identity	9

7.8 Optional connected-CNC cycle counting	10

7.9 Cimatron source pattern for cycle blocks	10

7.10 Test examples before delivery	10

7.11 What to deliver to Meimad	11

7.12 Compact Cimatron reference examples	11

Where to add the hook	11

Cimatron post parameters	12

Important Cimatron split-output warning	12

Cimatron cycle events	12

Cimatron delivery check	12

Cimatron original GPP example	12

Cimatron GPP2 example with validation	13

Cimatron cycle-output reminder	13

8. Complete normal NC example	14

9. The Offset Loader call	14

10. Who writes each DPRNT event	14

11. Optional CST and CEN cycle events	15

Cycle start	15

Cycle end	15

Simple placement example	16

12. Easy CST/CEN rules	16

13. DPRNT text rules	16

14. What not to put in the post	17

15. Common errors	17

16. Final programmer checklist	18

17. Copy/paste quick reference	18

18. Quick postprocessor examples	18

SolidCAM GPPL - one main NC file	18

Cimatron original GPP - one main NC file	19

Cimatron GPP2 - validation and output	19

Expected generated NC from all three examples	20

19. References	20

# What you need to change

This guide explains the small changes needed in a Haas NGC postprocessor so its G-code works with Meimad.

You do not need to understand the Meimad Server, database, tablet, network security, or verification mathematics. You only need to generate the correct G-code lines in the correct places.

If you remember only five rules, remember these:

1. Write one Meimad verification call near the top of every released NC file.

2. It must be the first executable G-code line.

3. Give every new NC release a new six-digit NC ID.

4. Do not write OLC, SVS, or SVF DPRNT lines in the normal NC post. The protected macros write them.

5. Keep CST/CEN cycle DPRNT output disabled until Meimad approves the exact Machine setup.

# 1. The most important line

The post must write this line once:

G65 P9002 A817426. (MEIMAD VERIFY V1)

Meaning:

| Text | Simple meaning |
| --- | --- |
| G65 | Call a macro program. |
| P9002 | Call protected program O9002. The approved Machine setting may use another O9xxx number. |
| A817426. | The six-digit Meimad NC ID. The final decimal point is required in post output. |
| (MEIMAD VERIFY V1) | Fixed marker used by Meimad. Write it exactly like this. |

Do not copy 817426. It is only an example. Every newly released NC file needs a new ID between 100000 and 999999.

# 2. Where the line goes

The verification call must be the first executable line.

Comments, %, and the O-number may appear before it. Normal safety codes must appear after it.

Correct:

% O01994 (PART-100 OP10) (PART: PART-100) (OPERATION: 10) G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90

Wrong—G90 executes before verification:

% O01994 G90 G65 P9002 A817426. (MEIMAD VERIFY V1)

Also wrong—two verification calls:

G65 P9002 A817426. (MEIMAD VERIFY V1) G65 P9002 A817427. (MEIMAD VERIFY V1)

The complete NC file must contain exactly one (MEIMAD VERIFY V1) marker.

# 3. Values the post needs

Add these properties or inputs to the post:

| Post input | Required value |
| --- | --- |
| Meimad enabled | Yes/No. It must be Yes for a Meimad production release. |
| NC ID | A new number from 100000 to 999999. |
| Verify program | Normally 9002. Use the number approved for the target Machine. |
| Part | Part number or stable part name. |
| Case | Meimad Case name or number, when available. |
| Operation | Operation number/name, when available. |
| Revision | NC or part revision, when available. |
| Cycle DPRNT enabled | Default No. Enable only after Meimad commissioning. |
| Macro version | Needed only when cycle DPRNT is enabled. |

Stop post generation and show an error if Meimad is enabled but the NC ID or verification program is missing.

Do not automatically build the NC ID from the O-number, date, part number, or file name. These values can repeat. The NC ID must be new for each exact released file.

# 4. Header comments

Write simple identification comments before the verification call:

(PART: 456-123-A) (CASE: CASE-456) (OPERATION: 20) (REVISION: B)

Rules:

•  Write one clear PART: value.

•  Do not write two different part names in the header.

•  The O-number is not the Meimad NC ID.

•  The comments are useful to people, but the six-digit ID in the verification call identifies the exact NC release.

# 5. Simple post logic

The beginning of the post should work in this order:

write % write O-number write PART / CASE / OPERATION / REVISION comments check that the Meimad NC ID is six digits write the O9002 verification call write normal Haas safety codes write the rest of the normal NC program

Platform-independent pseudocode:

function writeMeimadVerification() { if meimadEnabled is false: return if ncId is not between 100000 and 999999: stop post with "Meimad NC ID must be six digits" if verifyProgram is not an approved O9xxx program: stop post with "Invalid Meimad verify program" write "G65 P" + verifyProgram + " A" + ncId + "." + " (MEIMAD VERIFY V1)" }

Call this function one time from the main program opening function. Do not call it from tool-change code, operation code, subprogram code, or the program footer.

# 6. Special instructions for SolidCAM post writers

SolidCAM posts use the GPPL language for G-code output. The .vmid file describes the Machine and controller/output settings, while the .gpp file contains the post logic. Add the Meimad output routine to the GPPL output logic. Do not put executable G-code into the Machine kinematics definition.

## Where to add the hook

Find the one GPPL procedure in your existing post that writes the beginning of the main NC output file. It normally writes %, the O-number, program comments, and then the Haas safety blocks.

Change that procedure to use this order:

write % and O-number write PART / CASE / OPERATION / REVISION comments call one shared GPPL routine that writes the Meimad O9002 hook write the normal Haas safety blocks

Do not add the hook to a SolidCAM job-start, tool-start, operation-start, or subprogram procedure that can run more than once. One generated NC file must contain exactly one hook.

Use the existing GPPL text/block output command from the post. The finished output line—not the GPPL source syntax—must be exactly this shape:

G65 P9002 A817426. (MEIMAD VERIFY V1)

If the post automatically adds N sequence numbers, an N number may appear before G65. Do not let the formatter add another address, remove the decimal point, change the fixed marker, or split the call across lines.

## SolidCAM properties

Provide the NC ID and Meimad enable switch as post inputs. A SolidCAM implementation may expose user-defined settings through the VMID/VMC and read them from GPPL, or may receive them from the controlled release workflow. Whichever method is used:

•  show the NC ID clearly to the CAM user;

•  validate that it contains exactly six digits;

•  never keep a sample NC ID as the production default;

•  default cycle DPRNT to disabled;

•  stop posting when required values are missing.

Keep the Meimad-enabled Haas post and its matching .gpp/.vmid files together. Test the pair actually selected by the CAM user; editing another post with a similar display name does not change the selected output.

## SolidCAM multi-job and subprogram warning

A SolidCAM project can contain many CAM jobs while still producing one main NC file. The verification hook belongs to the file, not to each job. If the post creates separate main files, each released file needs its own new NC ID and one hook. Do not reuse one NC ID automatically across multiple generated files.

Put optional CST immediately before the post's first counted machining output for one physical cycle. Put CEN only in the final-success branch for that cycle. Do not put CEN in a general end_of_job-style procedure if that procedure can also run after an incomplete or aborted path.

## SolidCAM delivery check

Deliver the edited .gpp, its matching .vmid, and generated test files. Regenerate G-code from SolidCAM and inspect the final file; do not accept a manually edited NC sample as proof that the GPPL change works.

## SolidCAM GPPL example

This example shows the normal integration shape used by many SolidCAM GPPL posts. Procedure names inside a real post may differ. Keep the existing file-opening helpers and insert the Meimad call between the header-comment helper and the first G/M-code helper.

; SOLIDCAM GPPL INTEGRATION EXAMPLE ; Assumed VMID variables: ; iVMID_MEIMAD_ENABLED ; iVMID_MEIMAD_NC_ID ; iVMID_MEIMAD_VERIFY_PROGRAM @start_of_file call @usr_sof_character call @usr_sof_progname call @usr_sof_commentsbeforecodes ; Add this one call here. call @usr_meimad_verification ; Existing Haas executable output must remain after Meimad. call @usr_sof_gmcodes call @usr_sof_commentsaftercodes endp @usr_meimad_verification if iVMID_MEIMAD_ENABLED eq 1 ; Validate the three VMID values with the error mechanism ; already used by this post before reaching this output. {nl,'G65 P'iVMID_MEIMAD_VERIFY_PROGRAM ' A'iVMID_MEIMAD_NC_ID '. (MEIMAD VERIFY V1)'} endif endp

The line break inside the GPPL source is only for document readability. If the local GPPL version does not allow the code-generation expression to span source lines, place the expression on one line:

{nl,'G65 P'iVMID_MEIMAD_VERIFY_PROGRAM' A'iVMID_MEIMAD_NC_ID'. (MEIMAD VERIFY V1)'}

The generated G-code must be one line:

G65 P9002 A817426. (MEIMAD VERIFY V1)

If the existing post writes G17 G21 G80 G90 inside @usr_sof_gmcodes, the placement above is correct. Do not put call @usr_meimad_verification after that helper.

Optional cycle routines can use the same GPPL output style, but the variables below are placeholders. Map them only after Meimad approves the persistent sequence source.

@usr_meimad_cycle_start if iVMID_MEIMAD_CYCLE_DPRNT eq 1 {nb,'DPRNT[MEIMAD/V/1/EVENT/CST/ID/'sMEIMAD_START_ID '/SEQ/'iMEIMAD_START_SEQ '/MACROVERSION/'iVMID_MEIMAD_MACRO_VERSION '/PROGRAM/'iVMID_MEIMAD_NC_ID']'} endif endp @usr_meimad_cycle_end if iVMID_MEIMAD_CYCLE_DPRNT eq 1 {nb,'DPRNT[MEIMAD/V/1/EVENT/CEN/ID/'sMEIMAD_END_ID '/SEQ/'iMEIMAD_END_SEQ '/MACROVERSION/'iVMID_MEIMAD_MACRO_VERSION '/PROGRAM/'iVMID_MEIMAD_NC_ID']'} endif endp

Call @usr_meimad_cycle_start once at the approved physical cycle-start boundary. Call @usr_meimad_cycle_end only from the complete-success branch. Do not place either call in @start_of_job or @end_of_job merely because the names look similar to cycle start/end.

# 7. Special instructions for Cimatron post writers

This section is written for a programmer who has never edited a Cimatron post. You need to make only two kinds of change:

1. Required: write one Meimad verification hook near the top of every main NC file.

2. Optional and Machine-commissioned: write one cycle-start event (CST) and one successful cycle-end event (CEN) for each physical part cycle.

Do not edit tool motion, feeds, speeds, offsets, work coordinates, or canned cycles for this integration.

## 7.1 First identify which Cimatron post you are using

Cimatron has two post systems. The names are similar, but their source files and syntax are not interchangeable.

| What you see | What it means | What to edit |
| --- | --- | --- |
| An original GPP post | The older GPP language and compiled post workflow | The editable original GPP source selected by the Post Process dialog |
| An EX2 program with a DF2 configuration | GPP2 | The matching EX2/DF2 pair selected for this Haas Machine |

For Cimatron 2026, GPP2 files are normally under the Cimatron Data\It\var\post2 area. Do not assume that a file with “Haas” in its name is active. Open Cimatron's Post Process dialog and write down the exact selected post before editing anything.

Beginner safety rule: copy the selected post and its configuration to a dated backup folder before editing. Compile or load only the copy intended for the test. Keep the known working production post available for immediate rollback.

## 7.2 Understand the output point

The Cimatron source is not the NC file. An OUTPUT statement in the source writes text into the NC file.

The hook belongs in the block that begins each new main NC file:

BEGINNING OF TAPE write % and O-number write comment-only header lines write the Meimad G65 hook <-- add it here write the first normal G/M code

Do not put the hook in BEGINNING OF PROC, beginning of toolpath, a tool-change block, a milling block, or a drilling block. Those blocks can run many times and would create duplicate hooks.

The generated NC file must look like this:

% O01995 (PART EXAMPLE) (REVISION A) G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90

The %, O-number, and full-line comments may be above the hook because they do not command the Machine. G17, G40, G49, G80, G90, tool calls, spindle commands, offsets, macro calls, and motion are executable. None of them may be above the hook.

## 7.3 Beginner workflow: make and prove the required change

1. In Cimatron, open the real job and note the exact post selected in Post Process.

2. Make a dated backup of that post and its associated configuration.

3. Open the editable source and search for BEGINNING OF TAPE.

4. Find the statements that output %, the O-number, and the first normal Haas G-code.

5. Add the NC ID input and range check shown below.

6. Add the Meimad OUTPUT statement immediately before the first normal Haas G-code.

7. Compile/reload the post using the customer's normal Cimatron procedure.

8. Post one simple procedure with split output disabled.

9. Open the generated NC file as plain text. Confirm that the hook occurs exactly once and is the first executable block.

10. Only after that test passes, test the customer's normal split-output choices and multi-procedure jobs.

Do not judge success from a clean compile alone. The generated NC text is the result that matters.

## 7.4 What each original GPP line means

In the original GPP example later in this section:

•  INTERACTION asks the operator/post writer for a value.

•  BEGINNING OF TAPE runs when a new main output file begins.

•  OUTPUT $ starts a new NC line.

•  " G65 P9002 A" is fixed text.

•  MEIMAD_NC_ID supplies the six-digit identity.

•  ". (MEIMAD VERIFY V1)" writes the required decimal point and comment.

The SEQUENCING format controls how the number is printed. Confirm that it produces exactly six digits with no sign, grouping separator, or decimal digits. Use the existing post's stop/error pattern to reject values below 100000 or above 999999. Never allow the default value 0 to be posted.

## 7.5 A simpler GPP2 example: verification is always required

Use this form when every file produced by the post must be a Meimad release:

FORMAT (SEQUENCING) MEIMAD_NC_ID ; INTERACTION (SEQUENCING) "Meimad NC ID - six digits" MEIMAD_NC_ID = 0 ; BEGINNING OF TAPE: IF (MEIMAD_NC_ID < 100000 ¦¦ MEIMAD_NC_ID > 999999) GPP_STOP "Enter the approved six-digit Meimad NC ID" ; END_IF; OUTPUT "% " \J "O" PGN ; // Existing comment-only header output stays here. OUTPUT $ " G65 P9002 A" MEIMAD_NC_ID ". (MEIMAD VERIFY V1)" ; // The existing first executable line follows.

GPP_STOP prevents an invalid release from being written. This is better than writing A0. or silently omitting the hook. GPP2 interaction values may remain selected between toolpath runs in the same session. Treat a remembered value only as a convenience; enter and verify the new approved six-digit NC ID for every new NC release.

## 7.6 Common placement mistakes

Wrong: the safety line executes before verification.

G17 G40 G49 G80 G90 G65 P9002 A817426. (MEIMAD VERIFY V1)

Correct:

G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90

Wrong: the hook is called from every procedure, so a three-procedure job receives three hooks. Correct: call it once from BEGINNING OF TAPE for that main output file.

Wrong: a script after the postprocessor inserts or moves lines after review. Correct: the final released bytes must still contain exactly one unchanged hook in the correct position. A post script runs after the output file is closed and must not be used to “repair” the hook.

## 7.7 Split output: every NC file needs a different identity

Cimatron can split output by group, toolpath folder, tool change, UCS, or procedure. Each generated main NC file is a separate release candidate.

Example: a job produces O1001, O1002, and O1003. The correct result is three newly approved IDs:

O1001 G65 P9002 A381204. (MEIMAD VERIFY V1) O1002 G65 P9002 A594816. (MEIMAD VERIFY V1) O1003 G65 P9002 A726305. (MEIMAD VERIFY V1)

Do not copy one ID into all three files. If the post receives only one ID but is about to create several main files, stop with a clear error or disable split output for that release. Inspect every generated file; checking only the first file is not enough.

## 7.8 Optional connected-CNC cycle counting

Do this only after Meimad supplies and approves the Machine-specific event-sequence variable and confirms Macro Version 10 for the Machine.

•  CST means one physical production cycle started. For a connected Machine, the first valid CST after Ready for Production moves the run to In Production.

•  CEN means that same physical cycle completed successfully and counts one completed part cycle.

•  A Cimatron procedure, toolpath, or tool change is not automatically a physical part cycle.

•  Output exactly one CST at the approved physical cycle start and exactly one CEN only on the normal successful completion path.

•  Do not output CEN from an alarm, abort, reset, optional-stop exit, or incomplete branch.

The following is the required generated Haas V10 pattern. Replace <EVENT_SEQUENCE_VARIABLE> with the approved persistent variable number and <NC_ID> with the approved six-digit identity. Do not type the angle brackets into the NC file.

(AFTER VERIFY HOOK AND BEFORE FIRST MACHINING ACTION) G103 P1 #30=ROUND[#<EVENT_SEQUENCE_VARIABLE>] IF [ABS[#<EVENT_SEQUENCE_VARIABLE>-#30] GT 0.0001] THEN #30=0. IF [#30 LT 0.] THEN #30=0. IF [#30 GE 899999.] THEN #30=0. #30=#30+1. #<EVENT_SEQUENCE_VARIABLE>=#30 DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-<NC_ID>-S-#3001[80]/SEQ/#30[60]/MACROVERSION/10/PROGRAM/<NC_ID>] G103 P0 (ON NORMAL COMPLETION IMMEDIATELY BEFORE M30) G103 P1 #30=ROUND[#<EVENT_SEQUENCE_VARIABLE>] IF [ABS[#<EVENT_SEQUENCE_VARIABLE>-#30] GT 0.0001] THEN #30=0. IF [#30 LT 0.] THEN #30=0. IF [#30 GE 899999.] THEN #30=0. #30=#30+1. #<EVENT_SEQUENCE_VARIABLE>=#30 DPRNT[MEIMAD/V/1/EVENT/CEN/ID/NC-<NC_ID>-E-#3001[80]/SEQ/#30[60]/MACROVERSION/10/PROGRAM/<NC_ID>] G103 P0 M30

The event sequence is evidence, not Machine identity or verification authority. Do not substitute Cimatron's N block numbers, the Haas parts counter, or #3001 for the approved persistent sequence variable.

## 7.9 Cimatron source pattern for cycle blocks

The exact procedure names differ between customer posts. Create one shared output routine for the sequence update and event record, then call it only from the approved physical boundaries. A simplified GPP2-style pattern is:

// Physical cycle start: after the G65 hook, before machining. CALL MEIMAD_WRITE_SEQUENCE_AND_CST ; // Existing toolpaths and machining output. // Successful completion branch only, immediately before M30. CALL MEIMAD_WRITE_SEQUENCE_AND_CEN ; OUTPUT $ " M30" ;

Do not copy these example procedure names blindly. Map them to valid syntax and the actual top-level flow in the customer's selected GPP or GPP2 post. Compare the resulting NC text with the exact Haas V10 pattern above.

## 7.10 Test examples before delivery

| Test | Expected result |
| --- | --- |
| One simple procedure, split disabled | One main file; one hook; hook is first executable block |
| NC ID 0, blank, five digits, or seven digits | Posting stops with a clear error |
| Three procedures in one main file | Still exactly one hook |
| Three tools in one main file | Still exactly one hook |
| Split output creates three main files | Three hooks total, one per file, each with a different approved ID |
| Post the same job again as a new release | Operator supplies a new approved ID; remembered value is not accepted blindly |
| Optional cycle counting enabled | One CST and one successful CEN per physical cycle, using Macro Version 10 |
| Optional cycle aborted before completion | CST may exist; CEN does not exist |

Review the Cimatron output log as well as the generated files. A warning-free log does not replace NC inspection.

## 7.11 What to deliver to Meimad

For original GPP, provide the editable source, the compiled post used for the test, the generated NC examples, and the Cimatron output log. For GPP2, provide the matching EX2/DF2 files, generated NC examples, and output log.

Also provide a short mapping note that names:

•  the exact selected Cimatron post;

•  the BEGINNING OF TAPE block changed;

•  the NC ID interaction variable;

•  whether split output is allowed and how unique IDs are supplied;

•  the exact physical CST and CEN call locations, if cycle counting is enabled;

•  the approved persistent event-sequence variable number;

•  the tested rollback post.

## 7.12 Compact Cimatron reference examples

Cimatron supports the original GPP system and GPP2. First identify which system the selected Haas post actually uses. Do not edit a GPP post when production selects GPP2, or the reverse.

•  Original GPP post logic is developed in the GPP program source and compiled for use by Cimatron.

•  GPP2 uses its post program and configuration files, including the EX2/DF2 environment.

•  In both systems, the requirement is the same: the final G-code file must contain one hook as its first executable block.

## Where to add the hook

Find the one top-level GPP or GPP2 block that writes the beginning of each main output file. Add one shared Meimad output procedure and call it after the O-number/header comments but before the first safety or motion block.

Do not call it from procedure-start, tool-change, drilling, milling, or other toolpath handlers. Those handlers may run many times.

Use the post's normal output statement to produce this final line:

G65 P9002 A817426. (MEIMAD VERIFY V1)

The exact GPP/GPP2 statement syntax depends on the existing post and Cimatron version. Copy the output style already used by that post for literal Haas blocks, but verify the final G-code text against this guide.

## Cimatron post parameters

Expose the Meimad NC ID, enable switch, verify-program number, and cycle-DPRNT switch as G-code/post parameters when practical. For GPP2, configuration-associated variables can be presented through the post interaction and read by the post program. Regardless of the UI method, the post must validate the value before writing the hook.

## Important Cimatron split-output warning

Cimatron can split posted output by group, toolpath folder, tool change, UCS, or procedure. Every resulting main NC file is a different release candidate and needs:

•  exactly one verification hook;

•  its own new six-digit NC ID;

•  the hook as that file's first executable block.

If the post receives only one NC ID but Cimatron is configured to create several files, stop posting with a clear error or disable split output for that release. Never place the same NC ID into every split file.

Also check any “script after postprocessor” step. It must not insert a second hook, move the hook, or change the final released bytes after review.

## Cimatron cycle events

If CST/CEN is later commissioned, add the calls to the top-level successful machining flow—not to every Cimatron procedure automatically. Cimatron procedure count does not necessarily equal physical part-cycle count. The post writer must use the Meimad-approved physical cycle boundary.

Place CEN only where the post knows the full physical cycle completed. Toolpath completion, one procedure completion, or reaching a common file footer is not sufficient by itself.

## Cimatron delivery check

For original GPP, deliver the editable source, compiled post used for the test, and generated G-code. For GPP2, deliver the matching program/configuration files and generated G-code. Run the post from Cimatron's actual Post Process dialog, inspect every output file when splitting is enabled, and review the Cimatron output log for warnings.

## Cimatron original GPP example

The following example follows the original GPP file structure. Add the declarations near the other FORMAT and INTERACTION statements. Add the output line inside the existing BEGINNING OF TAPE block.

* CIMATRON ORIGINAL GPP INTEGRATION EXAMPLE FORMAT (SEQUENCING) MEIMAD_NC_ID ; INTERACTION (SEQUENCING) "MEIMAD NC ID - SIX DIGITS" MEIMAD_NC_ID = 0 ; NEW_LINE_IS $ ; * Keep the post's existing NEW_LINE_IS statements here. BEGINNING OF TAPE: * Keep the existing initialization. OUTPUT "% " \J "O" PGN ; * Keep existing PART/CASE/OPERATION comment output here. * No executable G-code may be output yet. OUTPUT $ " G65 P9002 A" MEIMAD_NC_ID ". (MEIMAD VERIFY V1)" ; * The existing first Haas safety line now follows the hook. OUTPUT $ " G17 G40 G49 G80 G90" ;

Important: the SEQUENCING format controls how the number is printed. Confirm that MEIMAD_NC_ID produces exactly six digits with no sign, grouping separator, or decimal digits. The literal period after the variable creates the required Haas A817426. form. Add the range check using the same stop/error pattern already used by the customer's original GPP post.

## Cimatron GPP2 example with validation

GPP2 supports BOOLEAN_ interaction values and GPP_STOP, so the post can reject missing inputs before producing G-code.

// CIMATRON GPP2 INTEGRATION EXAMPLE FORMAT (SEQUENCING) MEIMAD_NC_ID MEIMAD_VERIFY_PROGRAM ; INTERACTION (BOOLEAN_) "Enable Meimad verification" MEIMAD_ENABLED = FALSE_ ; INTERACTION (SEQUENCING) "Meimad NC ID - six digits" MEIMAD_NC_ID = 0 ; INTERACTION (SEQUENCING) "Meimad verify O-program" MEIMAD_VERIFY_PROGRAM = 9002 ; BEGINNING OF TAPE: IF (MEIMAD_ENABLED == TRUE_) IF (MEIMAD_NC_ID < 100000 ¦¦ MEIMAD_NC_ID > 999999) GPP_STOP "Meimad NC ID must be 100000 through 999999" ; END_IF; IF (MEIMAD_VERIFY_PROGRAM < 9000 ¦¦ MEIMAD_VERIFY_PROGRAM > 9999) GPP_STOP "Meimad verify program must be O9000-O9999" ; END_IF; END_IF; OUTPUT "% " \J "O" PGN ; // Keep existing full-line header comments here. // Do not output any G/M block before the next statement. IF (MEIMAD_ENABLED == TRUE_) OUTPUT $ " G65 P" MEIMAD_VERIFY_PROGRAM " A" MEIMAD_NC_ID ". (MEIMAD VERIFY V1)" ; END_IF; // Existing executable initialization follows. OUTPUT $ " G17 G40 G49 G80 G90" ;

If Meimad files are mandatory for this post, remove the enable switch and always validate/output the hook. A production post must never silently generate an unverified release because a remembered interaction value was set to false.

## Cimatron cycle-output reminder

Use only the complete Haas V10 CST/CEN pattern in section 7.8. A single DPRNT line without its approved persistent sequence update is incomplete and must not be released.

# 8. Complete normal NC example

This is a complete no-motion example. It is useful for checking the post output without loading offsets or moving the Machine.

% O01995 (MEIMAD POST TEST) (PART: TESTSERVERVERIFICATION005) (CASE: TESTSERVERVERIFICATION005) (OPERATION: 10 TEST1) (REVISION: TEST) G65 P9002 A817426. (MEIMAD VERIFY V1) (NO MOTION) (NO OFFSET LOADS) (NO CYCLE DPRNT) G17 G40 G49 G80 G90 M30 %

Before creating a real release, replace 817426 with a new six-digit NC ID. Do not use the sample number.

# 9. The Offset Loader call

The Offset Loader is a separate NC file. It is not the normal machining program.

If your work includes the Offset Loader generator, write this call only after all offsets were written and checked successfully:

G65 P9001 A483920. B817426. M30

Meaning:

| Text | Simple meaning |
| --- | --- |
| P9001 | Call the protected Offset Loader challenge macro O9001. |
| A483920. | Six-digit Offset Loader token supplied by the Meimad Server. |
| B817426. | The same NC ID used by the main machining program. |
| M30 | Normal end after the successful call. |

Rules:

•  Both numbers must have a decimal point.

•  Do not invent the Offset Loader token.

•  Do not put this call in the normal machining post.

•  Do not reach this call after an offset write/readback failure.

•  Put it near the end of the successful Offset Loader path, immediately before M30.

Example shape:

% O01993 (MEIMAD OFFSET LOADER) (OFFSET WRITE COMMANDS) (OFFSET READBACK CHECKS) (IF ANY CHECK FAILS, ALARM AND DO NOT CONTINUE) G65 P9001 A<OFFSET_TOKEN>. B<NC_ID>. M30 %

The final generated file must contain real six-digit numbers, not angle-bracket placeholders.

# 10. Who writes each DPRNT event

DPRNT sends one text line from the Haas control to the Meimad CNC communication service.

The normal post does not write every Meimad event.

| Event | Meaning | Who writes it? |
| --- | --- | --- |
| OLC | Offset Loader completed | Protected O9001 macro. Not the normal post. |
| SVS | Verification succeeded | Protected verification macros. Not the normal post. |
| SVF | Verification failed | Protected verification macros. Not the normal post. |
| CST | Production cycle started | Normal post, only after commissioning. |
| CEN | Production cycle completed | Normal post, only after commissioning. |

The normal post must not print fake success or failure messages. Never add these lines to the normal post:

EVENT/OLC EVENT/SVS EVENT/SVF

There are other Meimad workflow events, but they are not postprocessor output. The post writer does not need to generate them.

# 11. Optional CST and CEN cycle events

Most post writers should initially leave this feature off.

Enable CST/CEN only when Meimad gives you:

•  confirmation that the Machine uses Macro Version 10;

•  the approved persistent event-sequence variable;

•  the exact cycle-start position;

•  the exact successful cycle-end position;

•  the approved QC hold/release behavior.

If any value is missing, generate no CST and no CEN.

## Cycle start

CST means “this physical machining cycle has started.” Put it immediately before the first counted machining action, after verification and any required QC approval.

Example record after the required sequence update:

DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-817426-S-#3001[80]/SEQ/#30[60]/MACROVERSION/10/PROGRAM/817426]

## Cycle end

CEN means “this physical machining cycle completed successfully.” Put it only on the normal successful path, after all required machining and checks.

Example record after the next required sequence update:

DPRNT[MEIMAD/V/1/EVENT/CEN/ID/NC-817426-E-#3001[80]/SEQ/#30[60]/MACROVERSION/10/PROGRAM/817426]

In this example:

•  the NC ID is 817426;

•  #30 carries the sequence value prepared immediately before each record;

•  #3001 contributes to a unique event ID;

•  the macro/interface version is 10.

Do not hard-code the sample NC ID. Use the complete sequence-update and DPRNT blocks in section 7.8; a bare DPRNT line is not a complete implementation.

## Simple placement example

This is only a placement map. The full runnable output pattern is in section 7.8.

G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90 (NORMAL SETUP, TOOLS, OFFSETS, AND PROBING) (APPROVED QC HOLD/RELEASE POINT) (WRITE APPROVED V10 SEQUENCE UPDATE AND CST BLOCK FROM SECTION 7.8) (ALL COUNTED MACHINING FOR ONE PHYSICAL CYCLE) (WRITE APPROVED V10 SEQUENCE UPDATE AND CEN BLOCK FROM SECTION 7.8) M30

# 12. Easy CST/CEN rules

Follow these rules if cycle events are enabled:

1. Write one CST when one physical cycle starts.

2. Write one CEN only when that same cycle finishes successfully.

3. Update the approved persistent sequence once before CST and once before CEN. The Server records gaps, duplicates, resets, wraps, and out-of-order evidence but sequence continuity alone does not authorize or invalidate the run.

4. Do not write CEN after an alarm, Reset, E-stop, failed probe, failed tool check, or operator abort.

5. Do not put CEN in a shared footer that an error path can reach.

6. For a multi-output NC cycle, write one CST/CEN pair for the complete cycle, not one pair for every part output.

7. Do not try to repair missing or duplicated events in the post.

The safest source structure is to call writeCycleEnd() only from the one branch that proves the whole cycle completed.

Pseudocode:

if cycleDprntEnabled: update approved persistent event sequence write V10 CST write normal machining run all required completion checks if all work completed successfully and cycleDprntEnabled: update approved persistent event sequence write V10 CEN write M30

# 13. DPRNT text rules

The Server expects the fields in this exact order:

MEIMAD/V/1/EVENT/<CODE>/ID/<ID>/SEQ/<NUMBER>/MACROVERSION/<NUMBER>/PROGRAM/<NC_ID>

Keep it simple:

•  Use uppercase letters.

•  Do not add spaces inside the message.

•  Use / between fields.

•  Keep the field order exactly as shown.

•  Use only uppercase letters, digits, and hyphens in IDs.

•  Keep each complete message at or below 512 bytes.

•  Use a different event ID for every new event.

•  Use the same six-digit NC ID in PROGRAM.

Wrong—spaces and field order changed:

DPRNT[MEIMAD / EVENT / CST / V / 1 / SEQ / 201]

Correct shape:

DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-817426-S-#3001[80]/SEQ/#30[60]/MACROVERSION/10/PROGRAM/817426]

# 14. What not to put in the post

Do not add any of these items:

•  Machine secret or key;

•  verification response calculation;

•  nonce calculation;

•  protected macro variable numbers;

•  O9001, O9002, or O9003 program bodies;

•  direct OLC, SVS, or SVF DPRNT output;

•  Offset Loader token in the normal machining file;

•  automatic bypass when verification fails;

•  CST/CEN before the sequence method is approved.

The post calls the protected macros. It does not replace them.

# 15. Common errors

| Error | Fix |
| --- | --- |
| Verification line is after G90 or another G-code | Move it before every executable block. |
| Two hooks appear | Call the hook-writing function only once from the main program opening code. |
| NC ID has five digits | Stop generation and request a value from 100000 to 999999. |
| A817426 has no decimal point | Output A817426.. |
| Extra B value appears on the O9002 line | Remove it. O9002 receives only the NC ID in A. |
| O9001 call appears in the machining program | Move it to the separate Offset Loader generator. |
| Post prints SVS after the hook | Remove it. O9002 owns SVS/SVF output. |
| CEN is written before M30 on every path | Move it to the successful completion branch only. |
| Same NC ID is used for a changed release | Generate a new ID and regenerate the file. |

Meimad may report these release errors:

| Meimad error | Meaning |
| --- | --- |
| verification_hook_required | The file has no hook. |
| verification_hook_not_first | Executable code appears before the hook. |
| verification_hook_ambiguous | The marker appears more than once. |
| verification_hook_invalid | The syntax, program number, or ID is wrong. |
| verification_identity_reused | The NC ID was already used. Generate a new one. |

# 16. Final programmer checklist

Before delivering the post, check:

☐  The file writes PART, CASE, OPERATION, and REVISION comments.

☐  Exactly one O9002 verification call is present.

☐  It is the first executable line.

☐  The NC ID contains six digits and ends with a decimal point in the A argument.

☐  The fixed marker is (MEIMAD VERIFY V1).

☐  Normal Haas safety codes come after the hook.

☐  The normal post does not contain OLC, SVS, or SVF DPRNT lines.

☐  The normal post does not contain an O9001 Offset Loader call.

☐  CST/CEN is disabled unless Meimad approved all required inputs.

☐  If enabled, CEN can run only after one complete successful cycle.

☐  A changed NC release gets a new six-digit NC ID.

☐  A no-motion test file was generated and published successfully.

# 17. Copy/paste quick reference

Main machining program—write once as the first executable line:

G65 P<VERIFY_O9XXX> A<NEW_NC_ID>. (MEIMAD VERIFY V1)

Separate Offset Loader—write after successful offset readback:

G65 P<CHALLENGE_O9XXX> A<OFFSET_TOKEN>. B<NC_ID>.

Optional commissioned cycle start and end:

Use the complete Macro Version 10 sequence-update and DPRNT blocks in section 7.8.

Do not write in the normal post:

OLC SVS SVF

# 18. Quick postprocessor examples

These are short integration templates for a post writer. They are not complete vendor posts. Keep the existing post's file-opening, validation, formatting, and error functions, and adapt only the marked Meimad output point.

## SolidCAM GPPL - one main NC file

Add one call in the procedure that opens the main NC file. The call must be after comments and before the first normal Haas code:

@start_of_file call @usr_sof_character call @usr_sof_progname call @usr_sof_commentsbeforecodes ; MEIMAD: exactly once per generated main NC file. call @usr_meimad_verification call @usr_sof_gmcodes call @usr_sof_commentsaftercodes endp @usr_meimad_verification if iVMID_MEIMAD_ENABLED eq 1 {nl,'G65 P'iVMID_MEIMAD_VERIFY_PROGRAM' A'iVMID_MEIMAD_NC_ID'. (MEIMAD VERIFY V1)'} endif endp

Example inputs are iVMID_MEIMAD_ENABLED = 1, iVMID_MEIMAD_VERIFY_PROGRAM = 9002, and iVMID_MEIMAD_NC_ID = 817426. The final posted line must be:

G65 P9002 A817426. (MEIMAD VERIFY V1)

If the SolidCAM project creates two separate main NC files, call the routine once in each file-opening path and supply a different new six-digit NC ID for each file. Do not call it once per CAM job or tool.

## Cimatron original GPP - one main NC file

Declare the input with the other post interactions, then write it inside the existing BEGINNING OF TAPE block:

FORMAT (SEQUENCING) MEIMAD_NC_ID ; INTERACTION (SEQUENCING) "MEIMAD NC ID - SIX DIGITS" MEIMAD_NC_ID = 0 ; BEGINNING OF TAPE: OUTPUT "% " \J "O" PGN ; * Existing full-line header comments go here. * MEIMAD: this is the first executable block. OUTPUT $ " G65 P9002 A" MEIMAD_NC_ID ". (MEIMAD VERIFY V1)" ; * Existing Haas initialization follows. OUTPUT $ " G17 G40 G49 G80 G90" ;

Use the customer's existing GPP stop/error pattern to reject any value outside 100000 through 999999. Confirm that the local SEQUENCING format does not add a sign, grouping separator, or decimal digits.

## Cimatron GPP2 - validation and output

This compact GPP2 example stops posting when the NC ID is invalid, then writes the hook before ordinary Haas initialization:

FORMAT (SEQUENCING) MEIMAD_NC_ID ; INTERACTION (SEQUENCING) "Meimad NC ID - six digits" MEIMAD_NC_ID = 0 ; BEGINNING OF TAPE: IF (MEIMAD_NC_ID < 100000 ¦¦ MEIMAD_NC_ID > 999999) GPP_STOP "Meimad NC ID must be 100000 through 999999" ; END_IF; OUTPUT "% " \J "O" PGN ; // Existing full-line header comments go here. OUTPUT $ " G65 P9002 A" MEIMAD_NC_ID ". (MEIMAD VERIFY V1)" ; OUTPUT $ " G17 G40 G49 G80 G90" ;

For either Cimatron system, every split main output file needs one hook and its own new NC ID. If the post has only one NC ID input but produces several files, stop the post or disable split output for that release.

## Expected generated NC from all three examples

% O01995 (POST OUTPUT TEST) (PART: TESTSERVERVERIFICATION005) (OPERATION: 10 TEST1) G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90 M30 %

The sample ID 817426 is for documentation only. Assign a new six-digit NC ID before every real Meimad release.

# 19. References

•  Meimad API contract (api-contract.md)

•  Production Run architecture (production-run-architecture.md)

•  SolidCAM postprocessor overview (https://www.solidcam.com/highlights/postprocessors)

•  Cimatron GPP introduction (https://help.cimatron.com/en/2026/Introduction_to_GPP.htm)

•  Cimatron GPP/GPP2 Post Process dialog (https://help.cimatron.com/en/2026/post_processor/post_gpp.htm)

•  Cimatron GPP post file structure (https://help.cimatron.com/en/2026/Post_Processor_Program_File_Structure.htm)

•  Cimatron GPP BEGINNING OF TAPE block (https://help.cimatron.com/en/2026/block_beginning_of_tape.htm)

•  Cimatron GPP2 EX2/DF2 Post tab (https://help.cimatron.com/en/2026/gpp2_df2_editor_dialog_post_tab.htm)

•  Cimatron INTERACTION statement (https://help.cimatron.com/en/2026/interaction.htm)

•  Cimatron post-script behavior (https://help.cimatron.com/en/2026/system_calls.htm)

•  Cimatron GPP2 OUTPUT statement (https://help.cimatron.com/en/2026/OUTPUT.htm)

•  Haas macro documentation (https://www.haascnc.com/service/online-operator-s-manuals/mill-operator-s-manual/mill---macros.html)

•  Haas DPRNT documentation (https://www.haascnc.com/service/troubleshooting-and-how-to/how-to/communication-with-external-devices---dprnt.html)