# Simple Meimad Guide for Haas Postprocessor Programmers

Meimad Production Planner

Plain-language instructions, exact G-code lines, examples, and checks

| Document | Simple Meimad Guide for Haas Postprocessor Programmers |
| --- | --- |
| Audience | Postprocessor programmers who do not need Meimad system knowledge |
| Status | Writer guide; optional cycle DPRNT remains disabled until Machine-specific approval |
| Revision | 1.2 |
| Date | 27 August 2026 |
| Source | docs/haas-ngc-postprocessor-and-macro-specification.md |

Safety notice

Start with the verification hook only. Do not enable CST/CEN output until Meimad supplies and approves the Machine-specific sequence and cycle-boundary inputs.

Contents

What you need to change	3

1. The most important line	3

2. Where the line goes	3

3. Values the post needs	4

4. Header comments	4

5. Simple post logic	4

6. Complete normal NC example	5

7. The Offset Loader call	5

8. Who writes each DPRNT event	6

9. Optional CST and CEN cycle events	6

Cycle start	6

Cycle end	7

Simple placement example	7

10. Easy CST/CEN rules	7

11. DPRNT text rules	8

12. What not to put in the post	8

13. Common errors	9

14. Final programmer checklist	9

15. Copy/paste quick reference	9

16. References	10

# What you need to change

This guide explains the small changes needed in a Haas NGC postprocessor so its G-code works with Meimad.

You do not need to understand the Meimad Server, database, tablet, network security, or verification mathematics. You only need to generate the correct G-code lines in the correct places.

If you remember only five rules, remember these:

1. Write one Meimad verification call near the top of every released NC file.

2. It must be the first executable G-code line.

3. Give every new NC release a new six-digit NC ID.

4. Do not write OLC, SVS, or SVF DPRNT lines in the normal NC post. The protected macros write them.

5. Keep CST/CEN cycle DPRNT output disabled until Meimad approves the exact Machine setup.

# 1. The most important line

The post must write this line once:

G65 P9002 A817426. (MEIMAD VERIFY V1)

Meaning:

| Text | Simple meaning |
| --- | --- |
| G65 | Call a macro program. |
| P9002 | Call protected program O9002. The approved Machine setting may use another O9xxx number. |
| A817426. | The six-digit Meimad NC ID. The final decimal point is required in post output. |
| (MEIMAD VERIFY V1) | Fixed marker used by Meimad. Write it exactly like this. |

Do not copy 817426. It is only an example. Every newly released NC file needs a new ID between 100000 and 999999.

# 2. Where the line goes

The verification call must be the first executable line.

Comments, %, and the O-number may appear before it. Normal safety codes must appear after it.

Correct:

% O01994 (PART-100 OP10) (PART: PART-100) (OPERATION: 10) G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90

Wrong—G90 executes before verification:

% O01994 G90 G65 P9002 A817426. (MEIMAD VERIFY V1)

Also wrong—two verification calls:

G65 P9002 A817426. (MEIMAD VERIFY V1) G65 P9002 A817427. (MEIMAD VERIFY V1)

The complete NC file must contain exactly one (MEIMAD VERIFY V1) marker.

# 3. Values the post needs

Add these properties or inputs to the post:

| Post input | Required value |
| --- | --- |
| Meimad enabled | Yes/No. It must be Yes for a Meimad production release. |
| NC ID | A new number from 100000 to 999999. |
| Verify program | Normally 9002. Use the number approved for the target Machine. |
| Part | Part number or stable part name. |
| Case | Meimad Case name or number, when available. |
| Operation | Operation number/name, when available. |
| Revision | NC or part revision, when available. |
| Cycle DPRNT enabled | Default No. Enable only after Meimad commissioning. |
| Macro version | Needed only when cycle DPRNT is enabled. |

Stop post generation and show an error if Meimad is enabled but the NC ID or verification program is missing.

Do not automatically build the NC ID from the O-number, date, part number, or file name. These values can repeat. The NC ID must be new for each exact released file.

# 4. Header comments

Write simple identification comments before the verification call:

(PART: 456-123-A) (CASE: CASE-456) (OPERATION: 20) (REVISION: B)

Rules:

•  Write one clear PART: value.

•  Do not write two different part names in the header.

•  The O-number is not the Meimad NC ID.

•  The comments are useful to people, but the six-digit ID in the verification call identifies the exact NC release.

# 5. Simple post logic

The beginning of the post should work in this order:

write % write O-number write PART / CASE / OPERATION / REVISION comments check that the Meimad NC ID is six digits write the O9002 verification call write normal Haas safety codes write the rest of the normal NC program

Platform-independent pseudocode:

function writeMeimadVerification() { if meimadEnabled is false: return if ncId is not between 100000 and 999999: stop post with "Meimad NC ID must be six digits" if verifyProgram is not an approved O9xxx program: stop post with "Invalid Meimad verify program" write "G65 P" + verifyProgram + " A" + ncId + "." + " (MEIMAD VERIFY V1)" }

Call this function one time from the main program opening function. Do not call it from tool-change code, operation code, subprogram code, or the program footer.

# 6. Complete normal NC example

This is a complete no-motion example. It is useful for checking the post output without loading offsets or moving the Machine.

% O01995 (MEIMAD POST TEST) (PART: TESTSERVERVERIFICATION005) (CASE: TESTSERVERVERIFICATION005) (OPERATION: 10 TEST1) (REVISION: TEST) G65 P9002 A817426. (MEIMAD VERIFY V1) (NO MOTION) (NO OFFSET LOADS) (NO CYCLE DPRNT) G17 G40 G49 G80 G90 M30 %

Before creating a real release, replace 817426 with a new six-digit NC ID. Do not use the sample number.

# 7. The Offset Loader call

The Offset Loader is a separate NC file. It is not the normal machining program.

If your work includes the Offset Loader generator, write this call only after all offsets were written and checked successfully:

G65 P9001 A483920. B817426. M30

Meaning:

| Text | Simple meaning |
| --- | --- |
| P9001 | Call the protected Offset Loader challenge macro O9001. |
| A483920. | Six-digit Offset Loader token supplied by the Meimad Server. |
| B817426. | The same NC ID used by the main machining program. |
| M30 | Normal end after the successful call. |

Rules:

•  Both numbers must have a decimal point.

•  Do not invent the Offset Loader token.

•  Do not put this call in the normal machining post.

•  Do not reach this call after an offset write/readback failure.

•  Put it near the end of the successful Offset Loader path, immediately before M30.

Example shape:

% O01993 (MEIMAD OFFSET LOADER) (OFFSET WRITE COMMANDS) (OFFSET READBACK CHECKS) (IF ANY CHECK FAILS, ALARM AND DO NOT CONTINUE) G65 P9001 A<OFFSET_TOKEN>. B<NC_ID>. M30 %

The final generated file must contain real six-digit numbers, not angle-bracket placeholders.

# 8. Who writes each DPRNT event

DPRNT sends one text line from the Haas control to the Meimad CNC communication service.

The normal post does not write every Meimad event.

| Event | Meaning | Who writes it? |
| --- | --- | --- |
| OLC | Offset Loader completed | Protected O9001 macro. Not the normal post. |
| SVS | Verification succeeded | Protected O9002 macro. Not the normal post. |
| SVF | Verification failed | Protected O9002 macro. Not the normal post. |
| CST | Production cycle started | Normal post, only after commissioning. |
| CEN | Production cycle completed | Normal post, only after commissioning. |

The normal post must not print fake success or failure messages. Never add these lines to the normal post:

EVENT/OLC EVENT/SVS EVENT/SVF

There are other Meimad workflow events, but they are not postprocessor output. The post writer does not need to generate them.

# 9. Optional CST and CEN cycle events

Most post writers should initially leave this feature off.

Enable CST/CEN only when Meimad gives you:

•  the approved macro/interface version;

•  the approved sequence-number source;

•  the exact cycle-start position;

•  the exact successful cycle-end position;

•  the approved QC hold/release behavior.

If any value is missing, generate no CST and no CEN.

## Cycle start

CST means “this physical machining cycle has started.” Put it immediately before the first counted machining action, after verification and any required QC approval.

Example format:

DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-817426-S-201/SEQ/201/MACROVERSION/5/PROGRAM/817426]

## Cycle end

CEN means “this physical machining cycle completed successfully.” Put it only on the normal successful path, after all required machining and checks.

Example format:

DPRNT[MEIMAD/V/1/EVENT/CEN/ID/NC-817426-E-202/SEQ/202/MACROVERSION/5/PROGRAM/817426]

In this example:

•  the NC ID is 817426;

•  the CST sequence is 201;

•  the CEN sequence is the next number, 202;

•  the macro/interface version is 5;

•  the two event IDs are unique.

Do not hard-code these sample values.

## Simple placement example

This is only a placement example. It is not ready to run because the sequence-generation method is not shown.

G65 P9002 A817426. (MEIMAD VERIFY V1) G17 G40 G49 G80 G90 (NORMAL SETUP, TOOLS, OFFSETS, AND PROBING) (APPROVED QC HOLD/RELEASE POINT) DPRNT[MEIMAD/V/1/EVENT/CST/ID/<START_ID>/SEQ/<N>/MACROVERSION/<VERSION>/PROGRAM/817426] (ALL COUNTED MACHINING FOR ONE PHYSICAL CYCLE) DPRNT[MEIMAD/V/1/EVENT/CEN/ID/<END_ID>/SEQ/<N_PLUS_1>/MACROVERSION/<VERSION>/PROGRAM/817426] M30

# 10. Easy CST/CEN rules

Follow these rules if cycle events are enabled:

1. Write one CST when one physical cycle starts.

2. Write one CEN only when that same cycle finishes successfully.

3. The CEN sequence number must equal the CST number plus one.

4. Do not write CEN after an alarm, Reset, E-stop, failed probe, failed tool check, or operator abort.

5. Do not put CEN in a shared footer that an error path can reach.

6. For a multi-output NC cycle, write one CST/CEN pair for the complete cycle, not one pair for every part output.

7. Do not try to repair missing or duplicated events in the post.

The safest source structure is to call writeCycleEnd() only from the one branch that proves the whole cycle completed.

Pseudocode:

if cycleDprntEnabled: prepare approved sequence N write CST using N write normal machining run all required completion checks if all work completed successfully and cycleDprntEnabled: write CEN using N + 1 write M30

# 11. DPRNT text rules

The Server expects the fields in this exact order:

MEIMAD/V/1/EVENT/<CODE>/ID/<ID>/SEQ/<NUMBER>/MACROVERSION/<NUMBER>/PROGRAM/<NC_ID>

Keep it simple:

•  Use uppercase letters.

•  Do not add spaces inside the message.

•  Use / between fields.

•  Keep the field order exactly as shown.

•  Use only uppercase letters, digits, and hyphens in IDs.

•  Keep each complete message at or below 512 bytes.

•  Use a different event ID for every new event.

•  Use the same six-digit NC ID in PROGRAM.

Wrong—spaces and field order changed:

DPRNT[MEIMAD / EVENT / CST / V / 1 / SEQ / 201]

Correct shape:

DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-817426-S-201/SEQ/201/MACROVERSION/5/PROGRAM/817426]

# 12. What not to put in the post

Do not add any of these items:

•  Machine secret or key;

•  verification response calculation;

•  nonce calculation;

•  protected macro variable numbers;

•  O9001 or O9002 program bodies;

•  direct OLC, SVS, or SVF DPRNT output;

•  Offset Loader token in the normal machining file;

•  automatic bypass when verification fails;

•  CST/CEN before the sequence method is approved.

The post calls the protected macros. It does not replace them.

# 13. Common errors

| Error | Fix |
| --- | --- |
| Verification line is after G90 or another G-code | Move it before every executable block. |
| Two hooks appear | Call the hook-writing function only once from the main program opening code. |
| NC ID has five digits | Stop generation and request a value from 100000 to 999999. |
| A817426 has no decimal point | Output A817426.. |
| Extra B value appears on the O9002 line | Remove it. O9002 receives only the NC ID in A. |
| O9001 call appears in the machining program | Move it to the separate Offset Loader generator. |
| Post prints SVS after the hook | Remove it. O9002 owns SVS/SVF output. |
| CEN is written before M30 on every path | Move it to the successful completion branch only. |
| Same NC ID is used for a changed release | Generate a new ID and regenerate the file. |

Meimad may report these release errors:

| Meimad error | Meaning |
| --- | --- |
| verification_hook_required | The file has no hook. |
| verification_hook_not_first | Executable code appears before the hook. |
| verification_hook_ambiguous | The marker appears more than once. |
| verification_hook_invalid | The syntax, program number, or ID is wrong. |
| verification_identity_reused | The NC ID was already used. Generate a new one. |

# 14. Final programmer checklist

Before delivering the post, check:

☐  The file writes PART, CASE, OPERATION, and REVISION comments.

☐  Exactly one O9002 verification call is present.

☐  It is the first executable line.

☐  The NC ID contains six digits and ends with a decimal point in the A argument.

☐  The fixed marker is (MEIMAD VERIFY V1).

☐  Normal Haas safety codes come after the hook.

☐  The normal post does not contain OLC, SVS, or SVF DPRNT lines.

☐  The normal post does not contain an O9001 Offset Loader call.

☐  CST/CEN is disabled unless Meimad approved all required inputs.

☐  If enabled, CEN can run only after one complete successful cycle.

☐  A changed NC release gets a new six-digit NC ID.

☐  A no-motion test file was generated and published successfully.

# 15. Copy/paste quick reference

Main machining program—write once as the first executable line:

G65 P<VERIFY_O9XXX> A<NEW_NC_ID>. (MEIMAD VERIFY V1)

Separate Offset Loader—write after successful offset readback:

G65 P<CHALLENGE_O9XXX> A<OFFSET_TOKEN>. B<NC_ID>.

Optional commissioned cycle start:

DPRNT[MEIMAD/V/1/EVENT/CST/ID/<START_ID>/SEQ/<N>/MACROVERSION/<VERSION>/PROGRAM/<NC_ID>]

Optional commissioned cycle end:

DPRNT[MEIMAD/V/1/EVENT/CEN/ID/<END_ID>/SEQ/<N_PLUS_1>/MACROVERSION/<VERSION>/PROGRAM/<NC_ID>]

Do not write in the normal post:

OLC SVS SVF

# 16. References

•  Meimad API contract (api-contract.md)

•  Production Run architecture (production-run-architecture.md)

•  Haas macro documentation (https://www.haascnc.com/service/online-operator-s-manuals/mill-operator-s-manual/mill---macros.html)

•  Haas DPRNT documentation (https://www.haascnc.com/service/troubleshooting-and-how-to/how-to/communication-with-external-devices---dprnt.html)

•  Haas Setting 23 (https://www.haascnc.com/service/codes-settings.type%3Dsetting.machine%3Dmill.value%3DS23.html)