# NC Postprocessor Writer Guide

Meimad Production Planner

Implementation-aligned CAM and CNC release handoff

| Document | NC postprocessor writer guide |
| --- | --- |
| Audience | CAM postprocessor writers, release engineers, CNC commissioning personnel |
| Status | Implementation-aligned handoff; not CNC commissioning approval |
| Revision | 1.0 |
| Date | 27 August 2026 |
| Source | docs/postprocessor-writer-guide.md |

Safety notice

Correct postprocessor output is necessary for release publication. It does not prove CNC safety, authorize machining, or approve the quarantined protected-verification macro candidates.

Contents

1. Purpose and status	3

2. Writer responsibility boundary	3

3. Mandatory generic verification hook	3

4. Required placement	4

5. NC identity lifecycle	4

6. Header metadata	5

7. File delivery requirements	5

8. Offset Loader and protected verification boundary	5

9. Optional cycle DPRNT events	6

10. Server validation results	6

11. Writer implementation pattern	6

12. Acceptance checklist	7

13. Related authority	7

# 1. Purpose and status

This document is the handoff contract for a CAM/postprocessor writer producing NC files for Meimad Production Planner. It describes what the generated NC file must contain so the Server can publish it as an immutable G-code release and later bind that exact release to a Production Run.

The Server implementation and tests are the authority for accepted syntax. The protected Haas verification macros remain physically uncommissioned and quarantined. Correct postprocessor output is necessary for release publication, but it does not prove that a CNC is safe to run or that its verification interlock is production-approved.

# 2. Writer responsibility boundary

The postprocessor writer owns:

•  the ordinary controller-correct NC program;

•  one Meimad verification hook as the first executable block;

•  a new six-digit NC identity for every exact release candidate;

•  a stable part identity in the NC header;

•  preservation of the selected CAM postprocessor's machine/controller semantics;

•  optional production-cycle DPRNT events only when a separate commissioned contract explicitly requires them.

The postprocessor writer does not own:

•  Planner Postprocessor record creation or Machine-to-Postprocessor assignment;

•  Offset Loader generation or release tokens;

•  Machine secrets, response calculation, nonce generation, or protected macro variables;

•  O9001/O9002 installation or protection;

•  insertion or modification of NC files by the Server;

•  QC, planning, quantity, or workflow authority;

•  CNC commissioning or permission to enable protected verification.

Meimad stores the selected Planner Postprocessor by its stable Server ID. The NC bytes do not contain that ID. The release operator must select the matching Planner Postprocessor when publishing the file.

# 3. Mandatory generic verification hook

Every newly approved NC release must contain exactly one version-1 hook. It must be the first executable block.

Preferred Haas subprogram form:

G65 P9002 A817426. (MEIMAD VERIFY V1)

Accepted generic form:

[N<sequence>] G65 P9xxx A<identity>[. or .0...] (MEIMAD VERIFY V1)[;]

Accepted custom-G-code form, only when the target Machine configuration explicitly declares the same alias:

G605 A817426. (MEIMAD VERIFY V1)

Custom aliases may be G1 through G999 except G65. G0 is invalid. A G65 hook must call P9000 through P9999.

The line is intentionally strict:

•  A is exactly six decimal digits from 100000 through 999999.

•  The optional decimal suffix may be a decimal point alone or zeros only.

•  The marker must be (MEIMAD VERIFY V1); spacing and letter case inside the marker are flexible.

•  An optional N sequence word may precede the G word.

•  No other G/M/address word, block-delete character, inline comment, or executable command may appear on the hook line.

•  The marker must not occur anywhere else in the file.

The invocation must match the verification configuration of every Machine supported by this postprocessor. For example, a file using G65 P9002 is not compatible with a Machine configured for a different protected verify program or a custom G-code alias.

# 4. Required placement

The Server ignores these lines when locating the first executable block:

•  blank lines;

•  a line containing only %;

•  a full-line parenthesized comment;

•  an O program header, optionally followed by one parenthesized comment.

The hook must be the next non-ignored line. It must precede safety lines such as G17, G21, G40, G80, G90, or G94, as well as tool changes, spindle commands, offsets, motion, probing, and DPRNT.

Correct example:

% O01994 (PART-100 OP10) (PART: PART-100) (CASE: CASE-100) (OPERATION: 10) (REVISION: C) N10 G65 P9002 A817426. (MEIMAD VERIFY V1) N20 G17 G40 G49 G80 G90 (NORMAL POSTED PROGRAM CONTINUES HERE) M30 %

Incorrect because a modal block executes first:

O01994 G90 G65 P9002 A817426. (MEIMAD VERIFY V1)

Incorrect because the marker appears twice:

O01994 G65 P9002 A817426. (MEIMAD VERIFY V1) G65 P9002 A817427. (MEIMAD VERIFY V1)

Incorrect because additional words are present on the hook block:

G65 P9002 A817426. B1. (MEIMAD VERIFY V1)

# 5. NC identity lifecycle

The six-digit A value identifies the exact immutable NC release, not the part, Operation, O-number, Machine, or reusable Manufacturing Program.

The writer must follow this lifecycle:

1. Generate a six-digit value in the inclusive range 100000–999999 for a new release candidate.

2. Insert it once in the mandatory hook.

3. Finish generating the file before approval; do not edit approved bytes in place.

4. Publish the exact file through Meimad.

5. Treat the identity as permanently consumed after successful publication.

6. If the Server returns verification_identity_reused, generate a different identity and regenerate the file.

7. Any change to posted NC bytes that requires a new approved release also requires a new identity, even when the part, O-number, Postprocessor, or manufacturing recipe is unchanged.

The current system has no pre-allocation endpoint for identity values. Use a high-quality random six-digit generator and let the Server perform the authoritative global uniqueness check. Do not derive the value only from an O-number, date, part number, revision number, or file name because those values repeat.

Never copy an identity from a sample. Values such as 654321, 483921, 817426, and 328934 in repository examples are already test/history values and are not available for a new release.

# 6. Header metadata

Emit a stable part identity near the top of the file. The preferred form is:

(PART: 456-123-A)

Recommended additional fields are:

(CASE: CASE-7) (OPERATION: 20) (REVISION: B)

The Server reads the initial header and accepts PART: or PART NAME: with : or =. It also recognizes the established Meimad CAM form:

O1000 (16E2509-7PSOFI-1_NC1)

In that form the parsed part is 16E2509-7PSOFI-1. Do not emit two different part identities in one header. Conflicting identities make the header invalid. The Machine-side part header/DPRNT observation, when valid, is authoritative for monitoring; the O-number alone is not part identity.

# 7. File delivery requirements

Deliver a non-empty, line-oriented plain-text NC file. ASCII or UTF-8 is recommended for predictable controller and Server handling.

Default accepted G-code extensions are:

.nc .tap .gcode .cnc .iso .mpf .spf

The default maximum G-code size is 100 MiB and may be changed by Server configuration. The Server stores the original name as metadata, computes the file hash, copies the file into Server-owned immutable storage, and never rewrites the uploaded NC content.

The postprocessor name presented in CAM should map one-to-one to the managed Planner Postprocessor used at release, for example HAAS_4X. A display name is not evidence of compatibility: Meimad evaluates the stable selected Postprocessor ID against the Machine's explicit supported-ID list.

# 8. Offset Loader and protected verification boundary

Do not generate the Server Offset Loader inside the ordinary CAM postprocessor. The Offset Loader is a separate immutable Server release created for one Production Run, Machine, approved NC release, and exact tool-table release. Its six-digit release token comes from the Server after the NC release exists.

The ordinary NC program receives only the stable verification hook. It must not contain:

•  an Offset Loader release-token placeholder;

•  a Machine secret or derived key;

•  a copied nonce or response code;

•  protected-variable mappings;

•  code that writes or clears Planner workflow state;

•  an attempt to bypass a failed or unavailable verification.

Machine-specific protected-program numbers and variables are commissioning configuration, not portable business rules. Do not hard-code them beyond the invocation selected for the approved Postprocessor/Machine contract.

# 9. Optional cycle DPRNT events

Do not add cycle DPRNT output by default. Add it only after Meimad supplies a separately reviewed, Machine-specific emission contract and the CNC connection has one Server-owned DPRNT reader.

When commissioned, the Server accepts strict ASCII records in this order:

MEIMAD/V/1/EVENT/<CODE>/ID/<SOURCE_EVENT_ID>/SEQ/<NONNEGATIVE_INTEGER>/MACROVERSION/<POSITIVE_INTEGER>[/RUN/<ID>][/PROGRAM/<ID>]

For production observations, <CODE> is CST at the accepted cycle boundary and CEN at completed cycle end. Prefer the six-digit NC identity in PROGRAM. The END must use the same source, match the open START, and have sequence START + 1. A valid cycle END advances all coupled outputs atomically; an orphan, duplicate, interrupted, or nonconsecutive sequence is retained as evidence and does not invent completion.

Do not design a sequence source, source-event ID, look-ahead barrier, or DPRNT macro from this summary alone. Those details require controller review and commissioning. The repository's current macro candidates are not production approval.

# 10. Server validation results

Publication fails with HTTP 422 validation_failed and one of these detail codes:

| Code | Meaning | Writer action |
| --- | --- | --- |
| verification_hook_required | No marker was found. | Emit one accepted hook. |
| verification_hook_not_first | Another executable block precedes the marker. | Move the hook before all executable output. |
| verification_hook_ambiguous | The marker occurs more than once. | Emit it exactly once. |
| verification_hook_invalid | Hook syntax, invocation, or identity range is invalid. | Regenerate the exact accepted form. |
| verification_identity_reused | The six-digit identity already belongs to another release. | Generate a new identity and regenerate the file. |

Other publication failures—such as invalid Postprocessor selection, unsupported extension, excessive file size, tool-table mismatch, or missing Edit Mode—belong to the release workflow and do not authorize changing the hook contract.

# 11. Writer implementation pattern

Recommended generation order:

start file emit optional % emit O program header emit full-line identification comments allocate new six-digit NC identity emit exactly one verification hook emit normal safety/modal setup emit tools, offsets, spindle, coolant, and motion emit optional commissioned cycle evidence emit M30 and optional closing % finalize bytes do not mutate the candidate after approval

Keep the hook emission in one shared postprocessor routine. Do not duplicate it in operation templates, tool-change routines, subprogram headers, or user-added start blocks.

# 12. Acceptance checklist

The postprocessor writer should provide one normal production sample and one no-motion test sample for every supported post configuration.

☐  The CAM post name and target controller are identified.

☐  The hook invocation matches the approved Machine verification configuration.

☐  The file contains exactly one (MEIMAD VERIFY V1) marker.

☐  The hook is the first executable block.

☐  The A value is a new six-digit identity from 100000 through 999999.

☐  No example or previously published identity was reused.

☐  The header has one unambiguous part identity.

☐  The file uses an accepted extension and plain-text encoding.

☐  Ordinary safety/modal output appears after the hook.

☐  Offset Loader tokens, secrets, nonces, protected variables, and response math are absent.

☐  Optional DPRNT output is absent unless separately commissioned.

☐  Meimad publishes the sample and reports the expected verificationHook metadata.

☐  A second release receives a different NC identity.

☐  A deliberately late, duplicate, malformed, and reused hook is rejected during bench validation.

☐  CNC execution is not enabled based only on Server or simulator tests.

# 13. Related authority

•  API contract (api-contract.md), G-code release and CNC verification sections

•  Production Run architecture (production-run-architecture.md), schema-v51 generic-hook decision

•  Haas protected-verification spike (haas-protected-verification-spike.md), physical acceptance boundary

•  CNC verification code audit (cnc-verification-code-audit-2026-08-27.md), current quarantine and redesign requirements

•  CNC commissioning checklist (cnc-commissioning-checklist.md), per-Machine acceptance evidence

•  User help (user-help.md), operator release workflow