NC postprocessor and Production Package specification

Source-template markers, package-build rules, and Haas NGC, FANUC 0i/30i/31i, Mazak Matrix and Okuma OSP-P200 examples with SolidCAM and Cimatron patterns

Revision: 1.9 (2026-09-18) Active protocol: canonical Production Package protocol v2 Controllers covered: Haas NGC; FANUC 0i-D/0i-F and 30i/31i/32i-B; Mazak Matrix / Matrix 2 (EIA); Okuma OSP-P200/P300 (User Task 2) Audience: SolidCAM/Cimatron postprocessor writers and CNC controls reviewers Supersedes: the Haas-only "Haas NGC postprocessor and Production Package specification" and Word guides v1.7 and v1.8

The postprocessor is server-blind. It writes normal deterministic cutting code for its control and exact Meimad placeholders. It does not contact the Planner, select a Machine, create a package, assign an NC identity, or copy Part/Operation names from CAM fields into Meimad metadata.

The authoritative contract is postprocessor-production-package-contract.md. The placeholder grammar is the same for every control. What differs per control is the surrounding syntax the writer emits (comments, program header, output statements) and the syntax the Server injects, which follows the NC dialect configured on the Machine in Setup: HAAS_NGC, FANUC_MACRO_B, MAZAK_MATRIX_EIA, or OKUMA_OSP (section 4).

Warning about the draft: a newer draft contract (adding a POSTPROCESSOR_ID token and other changes) exists at haas-ngc-postprocessor-and-macro-specification.v2-draft.md. That draft is not implemented by the current Production Package Creator. Do not write a postprocessor against it. POSTPROCESSOR_ID is not a recognized placeholder key; using it fails Production Package creation with production_package_placeholder_unknown. Use this document only.

# 1. Required canonical block

Every new CNC source template must contain this logical structure. Comment wording around a token may vary, but the token spelling may not, and the part-counting markers are part of the block, not an extra.

%

O1500

(PART: [[MEIMAD:PART_NAME]])

(OPERATION: [[MEIMAD:OPERATION_NAME]])

(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])

(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])

(MACHINE: [[MEIMAD:MACHINE_ID]])

(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])

(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])

[[MEIMAD:VERIFICATION_HOOK]]

[[MEIMAD:EVENT_CONTEXT]]

DPRNT[[[MEIMAD:PART_NAME]]]

[[MEIMAD:CYCLE_START]]

G90 G17 G40 G49 G80

(normal CAM-generated tools, motion, feeds, speeds and cycles for one physical part)

[[MEIMAD:CYCLE_END]]

M30

%

| Key | Occurrences | Where it goes |
| --- | --- | --- |
| PART_NAME | one or more | a ( ) header comment and the bare print line of section 2 |
| OPERATION_NAME | one or more | ( ) comments only, never inside a print statement |
| PRODUCTION_RUN_ID | exactly one | ( ) comment |
| PRODUCTION_PACKAGE_ID | exactly one | ( ) comment |
| MACHINE_ID | exactly one | ( ) comment |
| NC_RELEASE_ID | exactly one | ( ) comment |
| OFFSET_LOADER_RELEASE_ID | exactly one | ( ) comment |
| VERIFICATION_HOOK | exactly one | its own line, before the first executable block |
| EVENT_CONTEXT | exactly one | its own line, after the hook (after POPEN on FANUC) |
| CYCLE_START | exactly one | its own line, immediately before the first block of the physical cycle |
| CYCLE_END | exactly one | its own line, after the last block of the cycle on the successful path, before M30/M02 |

The hook must be before the first executable NC block; a DPRNT, PUT, POPEN, G, M, T, or S word is executable, a %, an O program header, an empty line, a full-line ( ) comment, or a placeholder line is not.

Do not emit a real Part name, Operation name, Planner Machine ID, Run ID, Package ID, release ID, user, timestamp, challenge, response, or verification call. The Server owns all of those values.

MACHINE_ID, NC_RELEASE_ID, RUN_ID (via PRODUCTION_RUN_ID), and PACKAGE_ID (via PRODUCTION_PACKAGE_ID) resolve to short unique 6-digit numbers meant to be read and typed by hand at the control (e.g. 483921), not to the Server's internal identifiers. Do not assume these resolved values are the same length or format as any internal ID string.

# 2. The PartName output line

The Server takes machine-side Part identity from one bare output line that contains only the Part number, emitted by the runnable NC at program start. On Haas, FANUC, and Mazak it is a DPRNT; on Okuma it is a PUT flushed by WRITE C:

DPRNT[[[MEIMAD:PART_NAME]]]

PUT '[[MEIMAD:PART_NAME]]'

WRITE C

After package resolution the control prints, for example, 30P647004101-001, and the Server accepts the line because it is part-number-shaped: uppercase letters and digits, at least one digit, at least one - or . separator between groups, and not ending in .CNC. Everything else on the DPRNT channel is ignored for Part identity.

| Printed line | Accepted as Part identity | Why |
| --- | --- | --- |
| 30P647004101-001 | yes | letters/digits, a - separator, has digits |
| 4711.02 | yes | digits with a . separator |
| PART=30P647004101-001 | no | = is not allowed; the whole line is ignored |
| ABC123 | no | no - or . separator |
| AB_12-1 | no | _ is not allowed |
| 30p647-1 | no | lowercase (FANUC-family DPRNT would not print it anyway) |
| O1500.CNC | no | ends in .CNC (file names are never Part numbers) |
| MEIMAD/V/2/CONTEXT/... | no (event line) | handled by event ingestion, not Part identity |

- DPRNT[PART=[[MEIMAD:PART_NAME]]] is wrong: the control prints PART=30P647..., which is not part-number-shaped and is silently ignored. Earlier versions of this document and of the contract showed that form; it never matched.

- Do not put OPERATION_NAME or any other value inside a print statement. Operation names may contain spaces and lowercase letters, which DPRNT on FANUC-family controls does not output reliably and which the Server would not read anyway. Keep them in ( ) comments.

- Part numbers that the print line must carry are therefore limited to A-Z, 0-9, -, and .; a Part number such as ABC123 (no separator) or AB_12 (underscore) is never recognized on this path and would be identified only through the optional NC header reader.

- The Server strips ASCII control codes from every line before parsing, so the DC2 and DC4 codes that FANUC emits with POPEN/PCLOS, and NUL padding from a serial bridge, do not corrupt the line.

The line is executable, so it comes after VERIFICATION_HOOK. On FANUC-family controls it must also come after POPEN (section 6.2).

# 3. Part counting: the cycle markers are required

The Planner counts produced parts from two printed events per physical cycle: CST (cycle start) and CEN (cycle end). The postprocessor marks where they belong:

[[MEIMAD:CYCLE_START]]

(normal CAM-generated tools, motion, feeds, speeds and cycles for one complete

physical part cycle)

[[MEIMAD:CYCLE_END]]

Every new template contains exactly one CYCLE_START and exactly one CYCLE_END, each on its own line, CYCLE_START first. This is not optional: an Operation whose template has no pair produces no part count, no cycle timing, and no interrupted-cycle evidence, and the Run has to be counted by hand. The Server rejects a template that has one marker without the other (production_package_cycle_marker_unpaired) or the pair out of order (production_package_cycle_marker_order_invalid), and it refuses to build a package from a template that has neither marker (production_package_cycle_marker_required). The check runs when a package is built, so an older release without the pair fails at its next package build and has to be re-released with the pair.

Package Creator expands the pair into the wire-format part-counting events in the Machine's dialect (section 5) only when the assigned Machine has Server Verification enabled. On a verification-disabled Machine both markers are silently removed, nothing is printed, and nothing is counted, but the template stays valid; when verification is enabled later the same released NC starts counting without being re-posted. There is no non-verification path for automated part counting.

## 3.1 Placement rules

- CYCLE_START goes immediately before the work that begins one physical cycle: in a CAM program that machines one part per run, right after the header block, before the first G/T/S word.

- CYCLE_END goes only on the common successful path after that cycle has fully completed: after the last cut, after any in-program probing or deburring that belongs to the part, before M30/M99/M02 or any successful return.

- Never on an alarm, reset, optional-stop, or failure path. An M00/M01 between tools is fine inside the pair; a branch that skips the rest of the part after an M00 must not reach CYCLE_END.

- One pair per template. Do not wrap every tool, CAM procedure, or subprogram; do not put a marker in a subprogram that is called more than once. The pair belongs to the main program around the atomic cycle the Server should count.

- One pair means one counted cycle per program run. If a program machines two fixtures in one run, the Server still counts one cycle per run; the events carry no per-part multiplier.

## 3.2 Placement examples

Single part, tools in sequence (the normal CAM case):

DPRNT[[[MEIMAD:PART_NAME]]]

[[MEIMAD:CYCLE_START]]

G90 G17 G40 G49 G80

T1 M06

(roughing)

T2 M06

(finishing)

M01

T3 M06

(drilling)

[[MEIMAD:CYCLE_END]]

M30

Main program calling subprograms (M98): the pair stays in the main program, the subprograms carry no Meimad tokens at all.

DPRNT[[[MEIMAD:PART_NAME]]]

[[MEIMAD:CYCLE_START]]

G90 G54

M98 P2001 (SIDE A)

G55

M98 P2001 (SIDE B)

[[MEIMAD:CYCLE_END]]

M30

In-program measurement that belongs to the part: CYCLE_END comes after it, because the part is not finished until the probe cycle has run.

(finish milling)

G65 P9810 Z50. (PROBE RETRACT)

G65 P9814 D30. (BORE MEASURE)

[[MEIMAD:CYCLE_END]]

M30

Wrong: a marker per tool (duplicate markers are rejected) or CYCLE_END before an optional stop that may end the program early.

T1 M06

[[MEIMAD:CYCLE_START]]   <- rejected: production_package_placeholder_duplicate

(roughing)

[[MEIMAD:CYCLE_END]]

T2 M06

[[MEIMAD:CYCLE_START]]

(finishing)

[[MEIMAD:CYCLE_END]]

M30

# 4. NC dialects: what the Server generates for each control

Every Machine in Setup has an NC dialect. It selects the syntax of everything the Server injects and the variable numbers the Machine's verification configuration accepts. The template in section 1 does not change with the dialect.

| Injected element | HAAS_NGC | FANUC_MACRO_B and MAZAK_MATRIX_EIA | OKUMA_OSP |
| --- | --- | --- | --- |
| Verification hook | G65 P9002 A654321. (MEIMAD VERIFY V1) | same as Haas | CALL O9002 PA=654321 (MEIMAD VERIFY V1) |
| Event context | (MEIMAD EVENT CONTEXT V2) then DPRNT[MEIMAD/V/2/CONTEXT/...] | same as Haas | (MEIMAD EVENT CONTEXT V2) then PUT 'MEIMAD/V/2/CONTEXT/...' and WRITE C |
| Cycle block variables | scratch #30, sequence #10504 (configured), timer #3001 | scratch #30, sequence #504 (configured), timer #3001 | sequence VC5 (configured); the event ID uses the sequence instead of a timer |
| Look-ahead barrier | G103 P1 before and G103 P0 after the block | none (register a non-buffered M-code on the control if event order matters) | none |
| Branching in the cycle block | IF [...] THEN | IF [...] THEN | IF [...] NMDS1 / GOTO NMDS2 with sequence names NMDS1, NMDS2, NMDE1, NMDE2 |
| Event print | DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-654321-S-#3001[80]/SEQ/#30[60]/...] | same as Haas | PUT 'MEIMAD/V/1/EVENT/CST/ID/NC-654321-S-', PUT VC5,6,0, PUT '/SEQ/', PUT VC5,6,0, PUT '/MACROVERSION/6/PROGRAM/654321', WRITE C |
| Offset Loader artifact | offset-loader/O01990.nc: %, O01990 (...), comments, G65 P9001 A<release>. B654321., M30, % | same as Haas | offset-loader/O1990.MIN: comments, CALL O9001 PA=<release> PB=654321, M02 (no O line, no %) |
| Verification variables (nonce, state, release token, sequence) | #10000-#10999 | #500-#999 | VC1-VC200 (enter the number only) |
| Response variable | Haas M109 #500-#549 or #10500-#10549 | #500-#999 | VC1-VC200 |

Which dialect a Machine in this factory needs:

| Machine | Control | NC dialect | Connection type in Setup | DPRNT source |
| --- | --- | --- | --- | --- |
| Haas VF-3SS, UMC-500SS, ST-25Y | Haas NGC | HAAS_NGC | Haas MDC or Haas MTConnect | TCP (Setting 263 port) |
| FANUC-controlled mills | 0i-D/0i-F, 30i/31i/32i-B | FANUC_MACRO_B | FANUC FOCAS | TCP via serial-to-Ethernet bridge, or FILE |
| Mazak Variaxis i-500 | Matrix 2 (EIA) | MAZAK_MATRIX_EIA | DPRNT only | FILE \\192.168.0.158\print\print.txt |
| Okuma Genos L200E-M | OSP-P200L | OKUMA_OSP | DPRNT only | TCP via bridge, or FILE |

Rules that follow from the tables:

- The event context block is emitted for verification-enabled and verification-disabled Machines alike; the hook and the cycle blocks only when verification is enabled.

- Changing a Machine's dialect is refused while its Server Verification is enabled, because the stored variable numbers belong to the old control. Disable, change the dialect, re-enter the mappings, re-enable.

- Legacy (MEIMAD PACKAGE ... V1) releases can only be built for HAAS_NGC; a Machine with another dialect rejects them with production_package_dialect_legacy_unsupported. Re-release the NC as a canonical template.

- The Server strips all whitespace from a MEIMAD/ line before parsing it, so numeric fields printed with leading blanks (FANUC with parameter 6001#1 = 0, Okuma PUT VC5,6,0) are read correctly.

- Every dialect other than Haas NGC has been implemented from the control makers' documented syntax and is covered by Server tests, but has not yet run on a physical control in this factory. Commission it exactly like the Haas macros: bounded no-motion test first, verification disabled until the challenge/verify/finalizer subprograms for that control exist and pass.

# 5. Generated runnable NC examples

Given the template in section 1, NC identity 654321, Run 483002, Package 483001, macro version 6, verify program O9002, Server Verification enabled, and the cycle markers around one cycle. The text the control receives is exactly what is shown; the only writer-controlled parts are the header comments and the cutting code.

## 5.1 HAAS_NGC (Machine 10, event-sequence variable #10504)

%

O1500

(PART: 30P647004101-001)

(OPERATION: OP20 MILL 5AX)

(RUN: 483002)

(PACKAGE: 483001)

(MACHINE: 10)

(NC RELEASE: 654321)

(OFFSET LOADER: <offset-loader-release-id>)

G65 P9002 A654321. (MEIMAD VERIFY V1)

(MEIMAD EVENT CONTEXT V2)

DPRNT[MEIMAD/V/2/CONTEXT/PACKAGE/483001/RUN/483002/MACHINE/10/NCRELEASE/654321/MACROVERSION/6/PROGRAM/654321]

DPRNT[30P647004101-001]

G103 P1

#30=ROUND[#10504]

IF [ABS[#10504-#30] GT 0.0001] THEN #30=0.

IF [#30 LT 0.] THEN #30=0.

IF [#30 GE 899999.] THEN #30=0.

#30=#30+1.

#10504=#30

DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-654321-S-#3001[80]/SEQ/#30[60]/MACROVERSION/6/PROGRAM/654321]

G103 P0

G90 G17 G40 G49 G80

(one complete physical cycle)

G103 P1

#30=ROUND[#10504]

IF [ABS[#10504-#30] GT 0.0001] THEN #30=0.

IF [#30 LT 0.] THEN #30=0.

IF [#30 GE 899999.] THEN #30=0.

#30=#30+1.

#10504=#30

DPRNT[MEIMAD/V/1/EVENT/CEN/ID/NC-654321-E-#3001[80]/SEQ/#30[60]/MACROVERSION/6/PROGRAM/654321]

G103 P0

M30

%

The generated Haas Offset Loader (offset-loader/O01990.nc) for the same package:

%

O01990 (MEIMAD PACKAGE OFFSET LOADER)

(PRODUCTION PACKAGE 12)

(PRODUCTION RUN 483002)

(BATCH OPERATION <id>)

(MACHINE 10)

(NC RELEASE 654321)

(OFFSET LOADER RELEASE <id>)

(MEASURED TOOL OFFSETS - VERIFICATION AND RELEASE BINDING)

G65 P9001 A483920. B654321.

M30

%

## 5.2 FANUC_MACRO_B (Machine 11, event-sequence variable #504)

From the FANUC template of section 6.2, which adds POPEN/PCLOS:

%

O1500

(PART: 30P647004101-001)

(OPERATION: OP20 MILL 5AX)

(RUN: 483002)

(PACKAGE: 483001)

(MACHINE: 11)

(NC RELEASE: 654321)

(OFFSET LOADER: <offset-loader-release-id>)

G65 P9002 A654321. (MEIMAD VERIFY V1)

POPEN

(MEIMAD EVENT CONTEXT V2)

DPRNT[MEIMAD/V/2/CONTEXT/PACKAGE/483001/RUN/483002/MACHINE/11/NCRELEASE/654321/MACROVERSION/6/PROGRAM/654321]

DPRNT[30P647004101-001]

#30=ROUND[#504]

IF [ABS[#504-#30] GT 0.0001] THEN #30=0.

IF [#30 LT 0.] THEN #30=0.

IF [#30 GE 899999.] THEN #30=0.

#30=#30+1.

#504=#30

DPRNT[MEIMAD/V/1/EVENT/CST/ID/NC-654321-S-#3001[80]/SEQ/#30[60]/MACROVERSION/6/PROGRAM/654321]

G90 G17 G40 G49 G80

(one complete physical cycle)

#30=ROUND[#504]

IF [ABS[#504-#30] GT 0.0001] THEN #30=0.

IF [#30 LT 0.] THEN #30=0.

IF [#30 GE 899999.] THEN #30=0.

#30=#30+1.

#504=#30

DPRNT[MEIMAD/V/1/EVENT/CEN/ID/NC-654321-E-#3001[80]/SEQ/#30[60]/MACROVERSION/6/PROGRAM/654321]

PCLOS

M30

%

The Offset Loader is the same O01990.nc program as for Haas (section 5.1). Note that nothing but the two G103 lines differs from the Haas output; the #504 sequence variable comes from the Machine's verification configuration.

## 5.3 MAZAK_MATRIX_EIA (Machine 12)

Identical to the FANUC output of section 5.2 apart from the header values, without the POPEN/PCLOS lines if the template does not emit them. The DPRNT lines land in print.txt on the control (section 6.3) and the Server tails that file.

## 5.4 OKUMA_OSP (Machine 13, event-sequence variable VC5)

(PART: 30P647004101-001)

(OPERATION: OP20 TURN)

(RUN: 483002)

(PACKAGE: 483001)

(MACHINE: 13)

(NC RELEASE: 654321)

(OFFSET LOADER: <offset-loader-release-id>)

CALL O9002 PA=654321 (MEIMAD VERIFY V1)

(MEIMAD EVENT CONTEXT V2)

PUT 'MEIMAD/V/2/CONTEXT/PACKAGE/483001/RUN/483002/MACHINE/13/NCRELEASE/654321/MACROVERSION/6/PROGRAM/654321'

WRITE C

PUT '30P647004101-001'

WRITE C

VC5=ROUND[VC5]

IF [VC5 LT 0] NMDS1

IF [VC5 GE 899999] NMDS1

GOTO NMDS2

NMDS1 VC5=0

NMDS2 VC5=VC5+1

PUT 'MEIMAD/V/1/EVENT/CST/ID/NC-654321-S-'

PUT VC5,6,0

PUT '/SEQ/'

PUT VC5,6,0

PUT '/MACROVERSION/6/PROGRAM/654321'

WRITE C

G15 H1

(one complete physical cycle)

VC5=ROUND[VC5]

IF [VC5 LT 0] NMDE1

IF [VC5 GE 899999] NMDE1

GOTO NMDE2

NMDE1 VC5=0

NMDE2 VC5=VC5+1

PUT 'MEIMAD/V/1/EVENT/CEN/ID/NC-654321-E-'

PUT VC5,6,0

PUT '/SEQ/'

PUT VC5,6,0

PUT '/MACROVERSION/6/PROGRAM/654321'

WRITE C

M02

The generated Okuma Offset Loader (offset-loader/O1990.MIN):

(MEIMAD PACKAGE OFFSET LOADER)

(PRODUCTION PACKAGE 12)

(PRODUCTION RUN 483002)

(BATCH OPERATION <id>)

(MACHINE 13)

(NC RELEASE 654321)

(OFFSET LOADER RELEASE <id>)

(MEASURED TOOL OFFSETS - VERIFICATION AND RELEASE BINDING)

CALL O9001 PA=483920 PB=654321

M02

## 5.5 The same template on a verification-disabled Machine

Whatever the dialect, the hook line and both cycle blocks disappear, the Offset Loader comment resolves to NOT_APPLICABLE, and the context line is still printed:

%

O1500

(PART: 30P647004101-001)

(OPERATION: OP20 MILL 5AX)

(RUN: 483002)

(PACKAGE: 483001)

(MACHINE: 10)

(NC RELEASE: 654321)

(OFFSET LOADER: NOT_APPLICABLE)

(MEIMAD EVENT CONTEXT V2)

DPRNT[MEIMAD/V/2/CONTEXT/PACKAGE/483001/RUN/483002/MACHINE/10/NCRELEASE/654321/MACROVERSION/6/PROGRAM/654321]

DPRNT[30P647004101-001]

G90 G17 G40 G49 G80

(one complete physical cycle)

M30

%

The MEIMAD/V/2/CONTEXT/... line is correlation evidence in the raw DPRNT log; the Server's event ingestion only parses MEIMAD/V/1/EVENT/... lines and records the context line as an unsupported-version line. It is harmless and informational.

# 6. Controller templates and setup

The canonical block does not change between controls. Each example shows the complete source template the postprocessor must produce for that control, the control setup that routes the printed lines to the Server, and the Meimad settings that receive them.

## 6.1 Haas NGC (VF-3SS, UMC-500SS, ST-25Y): dialect HAAS_NGC

Haas NGC does not require POPEN/PCLOS. Mill template:

%

O1500

(PART: [[MEIMAD:PART_NAME]])

(OPERATION: [[MEIMAD:OPERATION_NAME]])

(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])

(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])

(MACHINE: [[MEIMAD:MACHINE_ID]])

(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])

(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])

[[MEIMAD:VERIFICATION_HOOK]]

[[MEIMAD:EVENT_CONTEXT]]

DPRNT[[[MEIMAD:PART_NAME]]]

[[MEIMAD:CYCLE_START]]

G90 G17 G40 G49 G80

G54

T1 M06

S8000 M03

G43 H1 Z50.

(CAM toolpaths)

G53 G49 Z0.

[[MEIMAD:CYCLE_END]]

M30

%

The ST-25Y lathe template is the same block with lathe modal codes (G18 G40 G80, G50 S3000, G96, G97) in the cutting section; the Meimad lines are identical.

Control setup:

- Setting 261 (DPRNT Destination) = TCP Port; Setting 263 (DPRNT TCP Port) = the port entered in Meimad (default 8080). Alternatively Setting 261 = Disk with Setting 262 naming a file under a shared User Data folder, which Meimad reads as a FILE source.

- The Server is the TCP client; the control accepts one read-only connection.

Meimad: Machine NC dialect HAAS_NGC; Connection type Haas MDC or Haas MTConnect; DPRNT source TCP (controller IP, Setting 263 port) or FILE. Verification variables #10500-range as today. Verification, Offset Loader, and part counting are implemented for this control and remain bench-commissioning gated (see cnc-commissioning-checklist.md).

## 6.2 FANUC 0i-D / 0i-F and 30i / 31i / 32i-B: dialect FANUC_MACRO_B

FANUC requires POPEN before any DPRNT and PCLOS after the last one. POPEN is executable, so it follows the hook; keep one POPEN at the start and one PCLOS before M30 so every DPRNT in between, including the Server-injected ones, is inside the open channel.

%

O1500

(PART: [[MEIMAD:PART_NAME]])

(OPERATION: [[MEIMAD:OPERATION_NAME]])

(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])

(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])

(MACHINE: [[MEIMAD:MACHINE_ID]])

(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])

(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])

[[MEIMAD:VERIFICATION_HOOK]]

POPEN

[[MEIMAD:EVENT_CONTEXT]]

DPRNT[[[MEIMAD:PART_NAME]]]

[[MEIMAD:CYCLE_START]]

G90 G17 G40 G49 G80

G54

T1 M06

S8000 M03

G43 H1 Z50.

(CAM toolpaths)

G91 G28 Z0.

[[MEIMAD:CYCLE_END]]

PCLOS

M30

%

POPEN sends the DC2 control code and PCLOS sends DC4 on the serial line; the Server strips both before parsing, so the first and last lines are read correctly.

Control setup (typical values; confirm each number against the parameter manual of the specific control and option set):

| Parameter | Purpose | Typical value |
| --- | --- | --- |
| No. 0000 bit 1 (ISO) | Output code for DPRNT | 1 = ISO (ASCII) |
| No. 0020 (I/O CHANNEL) | Device used by DPRNT | 0 or 1 = RS-232-C port 1 (JD36A); 2 = port 2 (JD36B) |
| No. 0101 / 0111 | Stop bits and NUL handling for channel 1 / 2 | per bridge settings |
| No. 0102 / 0112 | Device type for channel 1 / 2 | the entry defined as "user"/generic RS-232 device |
| No. 0103 / 0113 | Baud rate code for channel 1 / 2 | e.g. 11 = 9600, 12 = 19200 |
| No. 6001 bit 1 (PRT) | Space output in DPRNT text | either; the Server removes blanks from event lines |
| No. 6001 bit 4 (CRO) | End-of-block after DPRNT | 0 = LF only or 1 = LF + CR; the Server accepts both |
| No. 3411–3420 | M-codes that stop look-ahead buffering | optional: place one before the cycle blocks if event timing must match motion |

The RS-232 port is wired to a serial-to-Ethernet bridge configured as a TCP server with matching baud rate, data bits, parity, and stop bits. If the control carries a Data Server / memory-card DPRNT destination instead (parameter 20 = 4, 5, or 17), the output lands in a file on the control, not on the network; use it only where that file is reachable as a UNC path, then configure the FILE source.

Meimad: Machine NC dialect FANUC_MACRO_B; Connection type FANUC FOCAS (controller IP, MAC, FOCAS port 8193, part counter parameter 6711 or 6712); DPRNT source TCP with DPRNT TCP host = the bridge's IP and DPRNT TCP port = its listening port, or FILE for a shared print file, or NONE for monitoring only. Verification variables in #500-#999 (for example nonce #501, response #505, state #502, release token #503, sequence #504). The generated hook and cycle blocks are valid custom macro B; the protected O9001–O9003 subprograms and the operator response entry must be written and commissioned for FANUC before verification is enabled, so keep it disabled until then.

## 6.3 Mazak Matrix / Matrix 2 (EIA/ISO programs): dialect MAZAK_MATRIX_EIA

Matrix executes custom-macro-style DPRNT[...], G65, and # variables in EIA mode. POPEN/PCLOS may be present or absent; whether the DPR14 file device needs them is confirmed during commissioning, and the Server tolerates the DC2/DC4 they would add.

%

O1500

(PART: [[MEIMAD:PART_NAME]])

(OPERATION: [[MEIMAD:OPERATION_NAME]])

(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])

(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])

(MACHINE: [[MEIMAD:MACHINE_ID]])

(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])

(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])

[[MEIMAD:VERIFICATION_HOOK]]

[[MEIMAD:EVENT_CONTEXT]]

DPRNT[[[MEIMAD:PART_NAME]]]

[[MEIMAD:CYCLE_START]]

G90 G17 G40 G49 G80

G54.1 P1

T1 M06

S8000 M03

G43 H1 Z50.

(CAM toolpaths, including the tilted-plane blocks the Variaxis post already writes)

G91 G28 Z0.

[[MEIMAD:CYCLE_END]]

M30

%

Control setup:

- Parameter DPR14 = 4 routes DPRNT output to the file C:\MC_sdg\print\print.txt on the control PC. Share that folder (for example as print) read/write for the Server service account; the Mazak Variaxis i-500 Matrix 2 in this factory is read at \\192.168.0.158\print\print.txt.

- No G-code deletes the file. The Server empties it according to the Empty DPRNT file policy; ON_OFFSET_LOADER keeps exactly one setup session in the file.

Meimad: Machine NC dialect MAZAK_MATRIX_EIA; Connection type DPRNT only (no MDC/MTConnect/FOCAS telemetry on this control); DPRNT source FILE, file path as above, clear policy ON_OFFSET_LOADER; verification variables in #500-#999 as for FANUC. The generated syntax is the FANUC macro B form without G103; the protected subprograms must be ported and commissioned on the Matrix before verification is enabled.

Mazak Smooth (SmoothX / SmoothAi) controls: the DPRNT-to-file parameter differs from DPR14 and has not been verified in this project; treat the Smooth Variaxis as "DPRNT only, FILE source, path to be confirmed on the control".

## 6.4 Okuma OSP-P200 / P300: dialect OKUMA_OSP

Okuma OSP has no DPRNT, no # variables, and no G65; User Task 2 prints with PUT (text in single quotes, or a variable with digits,decimals) and flushes a line with WRITE C, calls subprograms with CALL Onnnn PA=... PB=..., keeps common variables in VC1-VC200, and branches with IF [...] Nname / GOTO Nname. The Server injects exactly that (section 4, section 5.4). Lathe template for the Genos L200E-M:

(PART: [[MEIMAD:PART_NAME]])

(OPERATION: [[MEIMAD:OPERATION_NAME]])

(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])

(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])

(MACHINE: [[MEIMAD:MACHINE_ID]])

(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])

(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])

[[MEIMAD:VERIFICATION_HOOK]]

[[MEIMAD:EVENT_CONTEXT]]

PUT '[[MEIMAD:PART_NAME]]'

WRITE C

[[MEIMAD:CYCLE_START]]

G15 H1

G50 S3000

G96 S200 M03

T010101

(CAM toolpaths)

G00 X300. Z200.

[[MEIMAD:CYCLE_END]]

M02

Rules for this template:

- ( ) comments are valid OSP comments, so the header block resolves unchanged.

- Release the program body without the $NAME.MIN% transfer header line: the Server's validator treats that line as the first executable block, which puts it before VERIFICATION_HOOK and fails the release with verification_placeholder_not_first.

- Do not start the file with an O name line; in an OSP .MIN file that turns the following blocks into a subprogram. The generated Offset Loader follows the same rule.

- The PUT/WRITE output device (the RS-232 port, or a print file on the OSP-P Windows side) is selected in the OSP parameters. Route it to a serial-to-Ethernet bridge (Meimad DPRNT source TCP, DPRNT TCP host = the bridge) or to a file on a shared folder (DPRNT source FILE). Confirm on the control which statement spelling and device the installed OSP version offers; the Server's OSP output is centralized in one place (OkumaOspDialect) so a spelling correction is a one-line change.

- The protected subprograms O9001–O9003 (challenge, verify, finalizer) do not exist for OSP yet; they are written and commissioned on the Genos L200E-M before verification is enabled. The Server side is complete: hook, event context, cycle blocks, Offset Loader (O1990.MIN), and VC1-VC200 variable validation.

Meimad: Machine NC dialect OKUMA_OSP; Connection type DPRNT only (there is no Okuma telemetry adapter yet); DPRNT source TCP via bridge or FILE; verification variables as plain numbers 1–200 (for example nonce 1, response 2, state 3, release token 4, sequence 5), which the Server renders as VC1…VC5.

# 7. SolidCAM GPPL examples

SolidCAM posts are GPPL procedures: a procedure begins with @name and ends with endp. Output is written with brace blocks, not an output command. { nl, ... } emits its comma-separated items on a new line, and { nb, ... } emits them on a new numbered block (with the sequence N-number). String literals use single quotes, and a literal placed directly before a variable concatenates ({ nl, 'O' program_number }). Use { nl, ... } for every Meimad line, so each token lands on its own line with no N-number, and write the tokens as literal strings, never bound to job, part, or user fields. Source comments start with ;.

## 7.1 Haas, FANUC, and Mazak posts (macro-B controls)

; ---- Meimad canonical header: literal tokens, no CAM values ----

@start_program

{ nl, '%' }

{ nl, 'O' program_number }

{ nl, '(PART: [[MEIMAD:PART_NAME]])' }

{ nl, '(OPERATION: [[MEIMAD:OPERATION_NAME]])' }

{ nl, '(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])' }

{ nl, '(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])' }

{ nl, '(MACHINE: [[MEIMAD:MACHINE_ID]])' }

{ nl, '(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])' }

{ nl, '(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])' }

{ nl, '[[MEIMAD:VERIFICATION_HOOK]]' }

; FANUC family only: open the print channel after the hook

{ nl, 'POPEN' }

{ nl, '[[MEIMAD:EVENT_CONTEXT]]' }

{ nl, 'DPRNT[[[MEIMAD:PART_NAME]]]' }

{ nl, '[[MEIMAD:CYCLE_START]]' }

{ nl, 'G90 G17 G40 G49 G80' }

endp

; ---- tools, motion, cycles: the normal SolidCAM procedures, unchanged ----

@end_program

{ nl, '[[MEIMAD:CYCLE_END]]' }

; FANUC family only: close the print channel before M30

{ nl, 'PCLOS' }

{ nl, 'M30' }

{ nl, '%' }

endp

Omit the two POPEN/PCLOS lines in the Haas and Mazak posts. CYCLE_START is the last line of @start_program and CYCLE_END the first line of @end_program because, for a CAM-generated program, the whole program is one physical cycle; if the post emits an in-program probing procedure after the last tool, CYCLE_END stays after it.

## 7.2 Okuma OSP post

@start_program

{ nl, '(PART: [[MEIMAD:PART_NAME]])' }

{ nl, '(OPERATION: [[MEIMAD:OPERATION_NAME]])' }

{ nl, '(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])' }

{ nl, '(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])' }

{ nl, '(MACHINE: [[MEIMAD:MACHINE_ID]])' }

{ nl, '(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])' }

{ nl, '(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])' }

{ nl, '[[MEIMAD:VERIFICATION_HOOK]]' }

{ nl, '[[MEIMAD:EVENT_CONTEXT]]' }

; the OSP PUT text contains single quotes, so this literal is double-quoted

{ nl, "PUT '[[MEIMAD:PART_NAME]]'" }

{ nl, 'WRITE C' }

{ nl, '[[MEIMAD:CYCLE_START]]' }

{ nl, 'G15 H1' }

endp

@end_program

{ nl, '[[MEIMAD:CYCLE_END]]' }

{ nl, 'M02' }

endp

No %, no O name line, M02 at the end, and the $NAME.MIN% transfer header is added by the transfer tool, never by the post (section 6.4).

## 7.3 Notes for the SolidCAM writer

- Procedure names vary between SolidCAM versions and machine definitions (@start_program/@end_program, @start_of_file/@end_of_file); the required result is the literal NC of section 1, whichever procedures produce it.

- Keep program_number, tool, and coordinate formatting exactly as the existing post defines them; Meimad does not read them.

- Do not use part_name, job or user text, or any SolidCAM variable in place of a Meimad token, and do not add HTTP, database, environment, or file lookups to the post.

- GPPL string literals use single quotes. The tokens contain only [, ], :, _, and letters, which are literal inside a GPPL string. When the line's own text contains a single quote, as in the Okuma PUT '...', write that literal with double quotes instead ({ nl, "PUT '[[MEIMAD:PART_NAME]]'" }).

- { nl, ... } outputs a line with no sequence number, which is what every Meimad header, hook, and print line needs. Use { nb, ... } only where the post already adds N-numbers to real motion blocks; do not put an N-number on a Meimad token line.

# 8. Cimatron GPP examples

Cimatron GPP posts are .exf execution files made of named blocks. The tape-level blocks are BEGINNING OF TAPE: (once, at the start of the NC file) and END OF TAPE: (once, at the end). OUTPUT \J "text" ; writes a literal on a new line with no sequence number; OUTPUT $ "text" ; writes a new line that does get one (used for the real N-numbered motion blocks, never for a Meimad token). OUTPUT "text" ; without \J or $ continues the current line. String literals are double-quoted, items concatenate by a space between them, and every statement ends with ;. A .exf source comment line starts with *, not with { } or //. Emit the tokens before any executable modal or motion block.

## 8.1 Haas, FANUC, and Mazak posts (macro-B controls)

BEGINNING OF TAPE:

OUTPUT "%" ;

OUTPUT \J "O1500" ;

OUTPUT \J "(PART: [[MEIMAD:PART_NAME]])" ;

OUTPUT \J "(OPERATION: [[MEIMAD:OPERATION_NAME]])" ;

OUTPUT \J "(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])" ;

OUTPUT \J "(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])" ;

OUTPUT \J "(MACHINE: [[MEIMAD:MACHINE_ID]])" ;

OUTPUT \J "(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])" ;

OUTPUT \J "(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])" ;

OUTPUT \J "[[MEIMAD:VERIFICATION_HOOK]]" ;

* FANUC family only: open the print channel after the hook

OUTPUT \J "POPEN" ;

OUTPUT \J "[[MEIMAD:EVENT_CONTEXT]]" ;

OUTPUT \J "DPRNT[[[MEIMAD:PART_NAME]]]" ;

OUTPUT \J "[[MEIMAD:CYCLE_START]]" ;

OUTPUT \J "G90 G17 G40 G49 G80" ;

* TOOL CHANGE:, LINEAR MOTION:, CIRCULAR MOTION:, cycles: the existing post blocks, unchanged

END OF TAPE:

OUTPUT \J "[[MEIMAD:CYCLE_END]]" ;

* FANUC family only: close the print channel before M30

OUTPUT \J "PCLOS" ;

OUTPUT \J "M30" ;

OUTPUT \J "%" ;

Omit the POPEN/PCLOS statements in the Haas and Mazak posts. The program number line uses whatever the existing post already outputs (a literal, or the post's program-number variable); Meimad does not read it.

## 8.2 Okuma OSP post

BEGINNING OF TAPE:

OUTPUT "(PART: [[MEIMAD:PART_NAME]])" ;

OUTPUT \J "(OPERATION: [[MEIMAD:OPERATION_NAME]])" ;

OUTPUT \J "(RUN: [[MEIMAD:PRODUCTION_RUN_ID]])" ;

OUTPUT \J "(PACKAGE: [[MEIMAD:PRODUCTION_PACKAGE_ID]])" ;

OUTPUT \J "(MACHINE: [[MEIMAD:MACHINE_ID]])" ;

OUTPUT \J "(NC RELEASE: [[MEIMAD:NC_RELEASE_ID]])" ;

OUTPUT \J "(OFFSET LOADER: [[MEIMAD:OFFSET_LOADER_RELEASE_ID]])" ;

OUTPUT \J "[[MEIMAD:VERIFICATION_HOOK]]" ;

OUTPUT \J "[[MEIMAD:EVENT_CONTEXT]]" ;

OUTPUT \J "PUT '[[MEIMAD:PART_NAME]]'" ;

OUTPUT \J "WRITE C" ;

OUTPUT \J "[[MEIMAD:CYCLE_START]]" ;

OUTPUT \J "G15 H1" ;

END OF TAPE:

OUTPUT \J "[[MEIMAD:CYCLE_END]]" ;

OUTPUT \J "M02" ;

## 8.3 Notes for the Cimatron writer

- Do not substitute Cimatron PART_NAME, the procedure name, the document name, or user text for the Meimad name placeholders. Those CAM values may remain in unrelated CAM comments, but they are never Meimad authority.

- Block names above are those of the classic GPP .exf; a GPP2 (.ex2) post keeps the same OUTPUT \J "..." ; statement and block markers, so check the sample post shipped with your Cimatron version for the exact block names it uses.

- Never write a Meimad token with OUTPUT $. That form adds the block's N sequence number, which turns a token line into two tokens on the same physical line and fails release validation (verification_placeholder_not_first for the hook, or a malformed placeholder for any other key). OUTPUT \J is correct for every line shown above.

# 9. Common mistakes and what the Server does with them

| Written by the post | Correct form | Result |
| --- | --- | --- |
| DPRNT[PART=[[MEIMAD:PART_NAME]]] | DPRNT[[[MEIMAD:PART_NAME]]] | Release passes; the printed line is ignored and the Part is never identified |
| POPEN above [[MEIMAD:VERIFICATION_HOOK]] | hook first, then POPEN | Release rejected: verification_placeholder_not_first |
| $O1500.MIN% header line at the top (Okuma) | no transfer header in the released body | Release rejected: verification_placeholder_not_first |
| [[MEIMAD:POSTPROCESSOR_ID]] (from the draft) | remove it | Package creation fails: production_package_placeholder_unknown |
| [[MEIMAD:EVENT_CONTEXT]] twice | once | production_package_placeholder_duplicate |
| a CYCLE_START/CYCLE_END pair per tool | one pair around the whole cycle | production_package_placeholder_duplicate |
| CYCLE_START without CYCLE_END (or CYCLE_END in a subprogram file) | both in the main program | production_package_cycle_marker_unpaired |
| CYCLE_END above CYCLE_START | start first | production_package_cycle_marker_order_invalid |
| neither cycle marker | both, exactly once | production_package_cycle_marker_required |
| G65 P9002 A1234. (MEIMAD VERIFY V1) written by the post | [[MEIMAD:VERIFICATION_HOOK]] | Release rejected: active verification content |
| (PART: 30P647004101-001) typed in the post | (PART: [[MEIMAD:PART_NAME]]) | Passes validation, but the header lies; the Server never cross-checks it |
| SolidCAM Meimad line in a { nb, ... } block | { nl, '...' } | The block prefixes an N sequence number, so the token is not a standalone line (verification_placeholder_not_first) |
| SolidCAM output "..." (invented, not GPPL) | { nl, '...' } | The post does not compile; GPPL has no output command |
| DPRNT[[[MEIMAD:OPERATION_NAME]]] | Operation name in a ( ) comment only | Prints an unreadable line; ignored |

# 10. Validation failures

Package/release validation rejects:

- a missing required key;

- malformed [[MEIMAD:...]] syntax;

- an unknown key;

- duplicate unique keys;

- a hook not on a standalone line or after executable code (including after POPEN, a DPRNT/PUT, or an Okuma $NAME.MIN% transfer header line);

- active Meimad verification logic embedded in the source;

- any unresolved required token in generated runnable NC;

- a template with neither cycle marker (production_package_cycle_marker_required), CYCLE_START without a matching CYCLE_END (or the reverse), or CYCLE_END before CYCLE_START;

- a legacy (MEIMAD PACKAGE ... V1) release on a Machine whose dialect is not HAAS_NGC (production_package_dialect_legacy_unsupported).

Validation does not check that the template's own statements match the Machine's dialect; a Haas-style DPRNT in a template released to an OKUMA_OSP Machine passes validation and alarms on the control. The pre-release checklist of section 13 catches the rest before the first package build.

# 11. Compatibility and commissioning

The former exact (MEIMAD PACKAGE VERIFY/CYCLE ... V1) format remains a named compatibility parser for immutable historical Haas releases. Do not emit it from a new or edited postprocessor. Existing .docx and dated machine-test documents are historical evidence, not current postprocessor instructions.

Generated verification and Offset Loader code still requires the existing Machine-specific review and bounded no-motion commissioning on every control. This document does not declare any protected macro, subprogram, or controller interlock commissioned.

# 12. Troubleshooting failures seen in practice

- production_package_placeholder_unknown: Unknown Meimad placeholder '<KEY>' on line N. — the source template contains a [[MEIMAD:<KEY>]] token that is not in the key set of section 1 (most commonly POSTPROCESSOR_ID, which is not and has never been a valid key; see the draft warning above). Fix the postprocessor to stop emitting it, then release a corrected NC.

- production_package_placeholder_duplicate: Canonical NC template must contain exactly one [[MEIMAD:EVENT_CONTEXT]]. — the template emits EVENT_CONTEXT twice, or a cycle marker more than once. The implemented protocol requires exactly one occurrence of each; one CYCLE_START/CYCLE_END pair around the whole cycle.

- The Part never appears in monitoring although DPRNT is connected. — the printed line is not part-number-shaped: a PART= prefix, a lowercase or underscore character, a Part number without a -/. separator, or a .CNC suffix. Print the bare Part number (section 2) and check the raw diagnostics (GET /api/v1/machines/{id}/cnc-diagnostics) for what the control actually sent.

- Parts are produced but the Run count stays at zero. — in this order: the package was built before 2026-09-18 from a template without the CYCLE_START/CYCLE_END pair (rebuilding it now fails with production_package_cycle_marker_required; re-release the NC with the pair), the Machine's Server Verification is disabled (the markers are removed), or the CST/CEN lines never reach the Server (FANUC: PCLOS before the CEN block; Mazak: the print file is not shared or not polled; Okuma: PUT without WRITE C).

- The control alarms on G103, #10504, DPRNT, or G65 in a generated package. — the Machine's NC dialect in Setup is wrong for that control (a FANUC or Mazak with HAAS_NGC, or an Okuma with any macro-B dialect). Set the dialect, re-enter the verification variables for that control, rebuild the package.

- ncDialect: verification_enabled when saving a Machine. — disable Server Verification for that Machine before changing its dialect, then re-enter the variable mappings in the new control's range.

- FANUC: the first DPRNT line after program start is missing or garbled. — an older Server did not strip the DC2 code that POPEN sends; upgrade the Server. If lines are still missing, the bridge is not in TCP-server mode or its serial settings do not match parameters 101–103.

- verification_placeholder_not_first on a release that looks correct — a non-comment line precedes the hook: POPEN, a DPRNT/PUT, a G/M/T/S word, or an Okuma $NAME.MIN% header. Move the hook above it or remove the line. In a SolidCAM post this is usually a missing "\J", which glued the hook to the previous line.

If a release keeps failing after a fix, use "View NC File (read-only)" or download the release directly and search for [[MEIMAD: to see every token and line number actually present, rather than assuming the macro emits what its source code says; a stale CAM export or wrong output folder can silently ship an older file.

# 13. Pre-release checklist for the writer

Run the post once on a test job and check the NC file, not the post source:

- Search for [[MEIMAD:; there are 12 tokens: PART_NAME twice (header comment and the print line), every other key exactly once.

- The first non-comment, non-%, non-O line is [[MEIMAD:VERIFICATION_HOOK]].

- [[MEIMAD:EVENT_CONTEXT]] is on its own line after the hook (after POPEN on FANUC).

- The print line is exactly DPRNT[[[MEIMAD:PART_NAME]]], or PUT '[[MEIMAD:PART_NAME]]' followed by WRITE C on Okuma, with nothing else on the line.

- [[MEIMAD:CYCLE_START]] precedes the first G/T/S word and [[MEIMAD:CYCLE_END]] follows the last cut on the successful path, before M30/M02; each once.

- No real Part number, Operation name, Machine number, or verification call appears anywhere the post wrote it.

- FANUC: POPEN after the hook, PCLOS after CYCLE_END. Okuma: no %, no O name line, no $NAME.MIN% header, M02 at the end.

- Release it to the Machine with the right NC dialect and build one package; open the generated runnable NC and compare it with section 5.

# 14. References

- PostProcessor -> Production Package Creator contract

- Postprocessor writer documentation audit, 2026-09-18

- Production Package implementation task

- Production Run architecture

- Haas verification response algorithm

- CNC commissioning checklist

- User help: CNC connection, DPRNT source, FANUC FOCAS